Table of Contents

Literature Review	3

Paper1:	3

Step 1: Comprehensive Executive Summary & Concept Breakdown	3

1. The Core Objective: What is this paper fighting against?	3

2. High-Level Engineering Intuition (Your 3rd-Year EE Breakdown)	4

3. Core Concepts & Mathematical Machinery Involved	4

Step 2: Literature Review Table	5

Step 3: Dataset Summary, Mathematical Modeling & Code Implementation	7

Paper2:	12

Step 1: Executive Summary & Conceptual Breakdown	12

Step 2: Comprehensive Literature Review Table	14

Step 3: Dataset Summary, Mathematical Modelling & Code Implementation	21

Paper 3	28

Step 1: Executive Summary & Conceptual Masterclass	28

Step 2: High-Precision Literature Review Table	30

Step 3: Dataset Characterization Summary	33

Core Mathematical Equations for Disturbances	33

1. Pure Normal Sinusoidal Line Voltage (Class C9)	34

y(t) = sin(ωt − φ)	34

2. Voltage Sag / Voltage Dip (Class C11)	34

y(t) = [1 − α(u(t − t1) − u(t − t2))] sin(ωt − φ)	34

Parameter Bounds: 0.1 ≤ α ≤ 0.9, and time window duration T ≤ (t2 − t1) ≤ 9T.	34

3. Voltage Swell / Voltage Surge (Class C14)	34

y(t) = [1 + α(u(t − t1) − u(t − t2))] sin(ωt − φ)	34

Parameter Bounds: 0.1 ≤ α ≤ 0.8, and time window duration T ≤ (t2 − t1) ≤ 9T.	34

4. Voltage Flicker (Class C1)	34

y(t) = [1 + λ sin(αωt)] sin(ωt − φ)	34

Parameter Bounds: Modulating depth 0.1 ≤ λ ≤ 0.2, modulating frequency 5 Hz ≤ α ≤ 50 Hz.	34

5. Voltage Harmonics (Class C5)	35

y(t) = sin(ωt − φ) + Σ Kn sin(nωt − φ)	35

n=3,5,7	35

Parameter Bounds: Harmonic distortion values 0.05 ≤ Kn ≤ 0.15, with normalization filter	35

Σ Kn² = 1	35

6. Complex Overlapping Class: Flicker + Sag (Class C3)	35

y(t) = [1 − α(u(t − t1) − u(t − t2))] · [1 + λ sin(αfωt)] · sin(ωt − φ)	35

7. Complex Overlapping Class: Flicker + Swell (Class C4)	35

y(t) = [1 + α(u(t − t1) − u(t − t2))] · [1 + λ sin(αfωt)] · sin(ωt − φ)	35

8. Complex Overlapping Class: Sag + Harmonics (Class C12)	35

y(t) = [1 − α(u(t − t1) − u(t − t2))] sin(ωt − φ)	35

+ Σ Kn sin(nωt − φ)	36

n=3,5,7	36

Python Generation Script for Research Baseline	36

Paper 4	38

Step 1: Executive Summary & Conceptual Masterclass (Paper 2)	38

Step 2: High-Precision Literature Review Table (Paper 2)	40

Step 3: Dataset Characterization Summary	44

Core Mathematical Equations for Disturbances	44

Python Script for Signal Re-creation & Simulation	46

Paper 5	47

Step 1: Executive Summary & Conceptual Masterclass	47

Step 2: High-Precision Literature Review Table	49

Step 3: Dataset Characterization Summary	52

Core Mathematical Equations for Disturbances	53

Python Generation Script: Creating the Trajectory Circle Shapes	54

Paper 6	56

Step 1: Executive Summary & Conceptual Breakdown	56

Step 2: Previewing the Literature Review & Dataset Sourcing	58

1. Data Sourcing and Preprocessing Findings	58

2. Signal Transformation Domain Findings	58

3. Feature Extraction and Selection Findings	59

4. Classification Techniques Benchmarking	59

5. Critical Research Gaps (Your Research Leads)	60

Step 2: High-Precision Literature Review Matrix	60

Step 3: Dataset Sourcing & Formula Engine	64

1. Dataset Sourcing & Availability Profile	64

2. Comprehensive Mathematical Formula Engine (Copy-Paste Format)	64

3. Executable Verification Waveform Code (Python 3)	66

Paper 7	69

1. Executive Summary & Main Findings	69

2. Literature Review Table	70

3. Dataset Summary & Generation Gist	70

Paper 8	71

1. Executive Summary & Main Findings	71

2. Literature Review Table	71

3. Dataset Summary & Generation Gist	72

Paper 3	72

1. Executive Summary & Main Findings	72

2. Literature Review Table	73

3. Dataset Summary & Generation Gist	74

Paper 9	74

1. Executive Summary & Main Findings	74

2. Literature Review Table	75

3. Dataset Summary & Generation Gist	75

Paper 10	76

Paper 11	78

Paper 12	79

Paper 13	81

Paper 14 Analysis	83

Paper 15:	85

Power Quality Disturbances Detection and Classification Based on Deep Convolution Auto-Encoder Networks	85

Paper 16:	86

Power Quality Disturbances Recognition via Lightweight Deep Learning Framework Integrated With Time-Frequency Analysis Embedded	86

Paper 17:	88

Perception of power quality disturbances using Fourier, Short-Time Fourier, continuous and discrete wavelet transforms	88

Paper 18:	89

Real-Time Detection and Classification of Power Quality Disturbances	89

# Literature Review

# Paper1:

## Step 1: Comprehensive Executive Summary & Concept Breakdown

: "Optimally detecting and classifying the transmission line fault in power system using hybrid technique" by P. Rajesh et al., 2022

### 1. The Core Objective: What is this paper fighting against?

When a fault happens on an electrical transmission line (like a short circuit caused by a tree branch or lightning strike), it creates severe Power Quality (PQ) disturbances—specifically rapid voltage drops (sags), spikes (swells), transient notches, and massive harmonic distortions.

Traditional protective relays struggle to analyze these fault waveforms in real time because the signals are highly non-stationary (they change instantly) and are buried under power grid noise. This paper introduces an automated system named TSVD-HUARPNN to instantly detect when a fault happens and classify exactly what kind of fault it is (e.g., Line-to-Ground, Line-to-Line) before it damages the wider grid.

### 2. High-Level Engineering Intuition (Your 3rd-Year EE Breakdown)

Think of a three-phase transmission line as a three-lane highway. When a crash (a fault) occurs, traffic patterns change instantly in all three lanes.

- The Problem with Traditional Methods: Methods like the Fast Fourier Transform (FFT) look at the average traffic over an hour. It can tell you there was a delay, but it cannot tell you the exact second the crash happened or which exact lanes are blocked.

- The Paper's Solution (The Matrix Trick): Instead of converting the signals into frequencies, the authors stack the measured 3-phase voltage and current waveforms over time into a giant data grid (a matrix). They use a mathematical tool called Truncated Singular Value Decomposition (TSVD) to strip away the noise and find the hidden structural "energy thumbprint" of the fault.

- The AI Brain: They pass these clean thumbprints into a specialized neural network called a Recurrent Perceptron Neural Network (RPNN). To make sure the neural network doesn't guess wrong, they use a optimization formula called the Human Urbanization Algorithm (HUA) which fine-tunes the network's brain cells to perform with extreme speed and accuracy.

### 3. Core Concepts & Mathematical Machinery Involved

To master this paper, we need to break down its three main engineering blocks:

#### A. Truncated Singular Value Decomposition (TSVD)

In linear algebra, any matrix A can be broken down into three matrices: A=UΣ(V^T)

- $\Sigma$ contains "singular values" which represent the strength of different features in your data.

- Standard SVD keeps all values, which includes random noise. Truncated SVD throws away the smallest singular values (treating them as meaningless noise) and only keeps the largest, most significant values. This compresses the transmission line's 3-phase voltage and current data into a clean, small set of features.

#### B. Recurrent Perceptron Neural Network (RPNN)

Standard neural networks take an input and give an output, treating every time step as completely independent. An RPNN has feedback loops (recurrence). Because an electrical fault is an evolving event (it starts, peaks, and clears over several cycles), the network needs "memory" to track how the signal changes from one millisecond to the next.

#### C. Human Urbanization Algorithm (HUA)

Training a neural network means finding the perfect mathematical weights. Instead of using standard trial-and-error (like gradient descent), the authors use a meta-heuristic optimization algorithm inspired by human migration patterns. It simulates how populations cluster into cities to systematically find the most optimal weights for the neural network, preventing it from getting stuck in slow or inaccurate classification paths.

## Step 2: Literature Review Table

| Parameter | Details Extracted from Paper 1 | Exact Location in the Paper |
| --- | --- | --- |
| Author(s) | P. Rajesh, R. Kannan, J. Vishnupriyan, B. Rajani | Page 1, Header/Title Block |
| Year | 2022 | Page 1, Article Info Metadata |
| Methodology | A hybrid framework named TSVD-HUARPNN. It uses Truncated Singular Value Decomposition (TSVD) to process three-phase voltage and current signal matrices into compact, noise-free singular value features, which are then fed into a Recurrent Perceptron Neural Network (RPNN) optimized by the Human Urbanization Algorithm (HUA). | Page 1, Abstract; detailed formulation across Section 2 (System Model) and Section 3 (Proposed Framework). |
| Dataset Used | Synthesized three-phase electrical power system transmission network data generated using MATLAB/Simulink. The model simulates a $400\text{ kV}$, $50\text{ Hz}$ transmission line network of $100\text{ km}$ length connected between two generation sources. | Section 4 (Results and Discussion), Page 8, Paragraph 1 & Table 1. |
| Features Extracted | Matrix representations of 3-phase voltages ($V_a, V_b, V_c$) and 3-phase currents ($I_a, I_b, I_c$) are decomposed using TSVD. The extracted features are the dominant singular values ($\sigma_i$) corresponding to the highest energy states of the signal matrix, filtering out lower-order singular values that represent high-frequency ambient noise. | Section 3.1 (Feature extraction using TSVD), Page 4, Equations 15–18. |
| Classification Technique | Recurrent Perceptron Neural Network (RPNN), whose structural parameters (interconnection weights and neuron thresholds) are optimally tuned using a meta-heuristic global search method called the Human Urbanization Algorithm (HUA). | Section 3.2 (Classifier) and Section 3.3 (HUA optimization workflow), Pages 5–7. |
| Performance Metrics | Classification Accuracy, Sensitivity (Recall), Specificity, Error Rate, and Computation Time (Execution latency). Evaluated under variations in fault inception angle, fault location ($10\text{ km}$ to $90\text{ km}$), fault resistance ($0.001\ \Omega$ to $100\ \Omega$), and Signal-to-Noise Ratios (SNR up to $50\text{ dB}$). | Section 4, Pages 8–11, explicitly illustrated in Tables 3, 4, 5, and Figures 9–14. |
| Limitations | 1. Highly complex mathematical pipeline due to the nested combination of matrix decomposition (TSVD) and meta-heuristic optimization (HUA). 2. Evaluation is constrained entirely to simulated environments (MATLAB/Simulink); lacking validation against real hardware-in-the-loop (HIL) or physical substation PMU (Phasor Measurement Unit) event log files. 3. Relies on fixed transmission line structural constants; adaptive performance under dynamically varying grid topologies is not evaluated. | Inferred from Section 4 constraints and Section 5 (Conclusion) on Page 11. |

## Step 3: Dataset Summary, Mathematical Modeling & Code Implementation

Dataset Name:

MATLAB/Simulink Transmission Grid Fault & Disturbance Dataset.

Source/Link:

Synthetically generated by the authors within a custom MATLAB/Simulink simulation platform (not a pre-existing public repository). Reference parameter profiles can be found in Section 4, Page 8, Table 1.

Number of Classes:

11 distinct operational conditions representing the transmission line states:

- 1 Healthy state (Normal system operation).

- 10 Fault conditions:

* Single line-to-ground faults: LG (Classes: AG, BG, CG)

* Line-to-line faults: LL (Classes: AB, BC, CA)

* Double line-to-ground faults: LLG (Classes: ABG, BCG, CAG)

* Three-phase symmetrical faults: LLL or LLLG

Sampling Frequency (fs):

Processed using standard grid framing. Transient window simulation settings approximate an effective collection speed between 1 kHz and 5 kHz to reliably register sub-cycle microsecond waveform points.

Number of Signals/Samples:

The synthesis matrix covers a highly diversified testing environment across the following permutations:

- Fault location: Varied from 10 km to 90 km (Step size = 10 km)

- Fault inception angle (theta): Varied from 0 degrees to 90 degrees (Step size = 15 degrees)

- Fault resistance (Rf): Tested at discrete steps of 0.001 Ohms, 10 Ohms, 25 Ohms, 50 Ohms, and 100 Ohms.

- Total Sample Count: Several thousand combined signal vectors compiled across all parameter permutations (Section 4.2).

#### 2. Core Mathematical Formulations

A. Normal Operating Three-Phase Fundamental Voltage Vectors:

Va(t) = Vm * sin(omega * t + theta)

Vb(t) = Vm * sin(omega * t + theta - 123.13_deg) = Vm * sin(omega * t + theta - 2*pi/3)

Vc(t) = Vm * sin(omega * t + theta + 123.13_deg) = Vm * sin(omega * t + theta + 2*pi/3)

Where:

- Vm is the peak phase-to-ground voltage amplitude calculation:

Vm = (400 kV * sqrt(2)) / sqrt(3)

- omega is the fundamental angular frequency:

omega = 2 * pi * f  (with system frequency f = 50 Hz)

- theta represents the specific fault inception angle.

B. Fault Modeling Boundary Mechanics (Section 2 & 3):

When an unexpected line fault initiates at time boundary t_fault, structural symmetry collapses.

For a Single Line-to-Ground (AG) Fault configuration, Phase A experiences ground path coupling governed by path resistance Rf:

Va(t) = Ia(t) * Rf + V_fault_ground

This configuration drops Va(t) to an amplitude dictated by the voltage drop factor while generating a steep transient current surge (Ia) via the low-impedance grounding route. Unaffected lines (Phases B and C) experience high-frequency modal coupling and transient distortion anomalies.

C. Spatiotemporal Multi-Phase Evaluation Matrix (A) passed into TSVD:

The raw monitored data samples are arranged by phase current and voltage rows over discrete time intervals (t1, t2, ... tn) to yield the composite tracking matrix A (Section 3.1, Equation 15):

|  Va(t1)   Vb(t1)   Vc(t1)   Ia(t1)   Ib(t1)   Ic(t1)  |

|  Va(t2)   Vb(t2)   Vc(t2)   Ia(t2)   Ib(t2)   Ic(t2)  |

A = |    .        .        .        .        .        .     |

|    .        .        .        .        .        .     |

|  Va(tn)   Vb(tn)   Vc(tn)   Ia(tn)   Ib(tn)   Ic(tn)  |

#### 3. Python Simulation Code (Transmission Grid Fault Waveforms)

import numpy as np

import matplotlib.pyplot as plt

# 1. Define Transmission Grid Simulation Parameters

f0 = 50                 # Base frequency (50 Hz)

fs = 2000               # Sampling frequency (2 kHz)

t_end = 0.3             # Total time window (300 ms)

t = np.arange(0, t_end, 1/fs)

# Peak voltage for a 400 kV system (Phase-to-Ground peak)

Vm = (400e3 * np.sqrt(2)) / np.sqrt(3)

omega = 2 * np.pi * f0

theta = np.radians(30)  # Fault inception angle of 30 degrees

# 2. Synthesize Normal Healthy Three-Phase Voltage States

Va = Vm * np.sin(omega * t + theta)

Vb = Vm * np.sin(omega * t + theta - (2 * np.pi / 3))

Vc = Vm * np.sin(omega * t + theta + (2 * np.pi / 3))

# 3. Inject a Line-to-Ground (AG) Fault Transient

t_fault_start = 0.10    # Fault hits at 100 ms

t_fault_end = 0.22      # Fault clears at 220 ms

# Create Boolean mask for the active fault period

fault_mask = (t >= t_fault_start) & (t <= t_fault_end)

# Phase A voltage experiences a 75% voltage drop (sag) during the ground fault

voltage_drop_factor = 0.25

Va[fault_mask] = Va[fault_mask] * voltage_drop_factor

# Fault locations also generate transient high frequency distortions (noise/harmonics)

# Let's add high frequency ringing artifact when the fault initiates

ringing = 0.08 * Vm * np.sin(15 * omega * t) * np.exp(-50 * (t - t_fault_start)) * (t >= t_fault_start)

Va += ringing

# 4. Construct the Data Matrix (as processed by TSVD in Section 3.1)

# Stacking Phase A, B, C over time

data_matrix = np.column_stack((Va, Vb, Vc))

# 5. Plot the Generated Waveforms

plt.figure(figsize=(11, 5))

plt.plot(t * 1000, Va / 1e3, label='Phase A (Faulted)', color='red', linewidth=1.5)

plt.plot(t * 1000, Vb / 1e3, label='Phase B (Healthy)', color='green', alpha=0.7)

plt.plot(t * 1000, Vc / 1e3, label='Phase C (Healthy)', color='blue', alpha=0.7)

plt.axvline(x=t_fault_start*1000, color='black', linestyle='--', label='Fault Inception')

plt.axvline(x=t_fault_end*1000, color='black', linestyle=':', label='Fault Clearance')

plt.title('Simulated Transmission Line Fault Waveforms (Phase-A-to-Ground Failure Condition)')

plt.xlabel('Time (milliseconds)')

plt.ylabel('Voltage (kV)')

plt.grid(True, linestyle='--', alpha=0.6)

plt.legend(loc='upper right')

plt.tight_layout()

plt.show()

print(f"Data Matrix Structure generated successfully. Shape: {data_matrix.shape}")

# Paper2:

A Critical Analysis of Methodologies for

Detection and Classification of Power

Quality Events in Smart Grid

## Step 1: Executive Summary & Conceptual Breakdown

1. Central Theme and Objective

This paper is a highly comprehensive critical review and comparative analysis

of the state-of-the-art methodologies utilized to detect, locate, and classify

Power Quality Disturbances (PQDs) within a Smart Grid paradigm. Rather than

introducing a single new algorithm, the authors systematically evaluate

dozens of existing frameworks. The core objective is to identify the open

bottlenecks, real-time computational trade-offs, and feature-extraction

vulnerabilities of various signal processing methods paired with artificial

intelligence.

2. Understanding the Smart Grid Paradigm Shift

In a traditional grid, power flow is unidirectional, and the generation

sources are synchronous and predictable. In a Smart Grid, we introduce:

- Bidirectional power flow.

- Massive penetration of Distributed Energy Resources (DERs) like solar PV

inverters and wind energy conversion systems.

- Non-linear, fast-switching power electronic converters and electric

vehicle (EV) charging stations.

These components introduce a highly unpredictable, non-linear environment,

escalating the occurrence of complex, overlapping, and non-stationary PQDs

(e.g., a voltage sag happening simultaneously with a high-frequency transient

and harmonic distortion).

3. The Three-Tier Architecture of PQ Automation

The paper emphasizes that any automated monitoring system operates on a

strict three-tier data pipeline:

Tier A: Data Acquisition / Pre-processing (filtering and windowing).

Tier B: Feature Extraction (the mathematical core that translates raw

time-domain signals into distinct statistical/spectral signatures).

Tier C: Classification (the machine learning or deep learning engine that

maps features to specific PQ event labels).

4. Detailed Critique of Mainstream Feature Extraction Tools

The authors break down the mathematical trade-offs of the dominant mathematical

transforms in the literature:

- Fast Fourier Transform (FFT): Highly efficient for pure stationary signals,

but completely loses time-domain resolution. It cannot identify when a transient

or sag begins or ends.

- Short-Time Fourier Transform (STFT): Fixes FFT's problem by using a fixed-width

sliding window. However, it suffers from Heisenberg's Uncertainty Principle—a

wide window gives good frequency resolution but poor time resolution; a narrow

window does the reverse. It fails on multi-frequency composite disturbances.

- Wavelet Transform (WT) / Discrete Wavelet Transform (DWT): Employs adjustable

multi-resolution analysis (narrow windows for high frequencies, wide windows

for low frequencies). It is excellent for sharp transients but highly sensitive

to background noise, and choosing the optimal "Mother Wavelet" is heavily heuristic.

- S-Transform (ST): A hybrid combination of STFT and WT that offers a

frequency-dependent window while retaining absolute phase data. It provides an

excellent time-frequency matrix but suffers from a very high computational load

(O(N^2 log N)), hindering low-cost microcontrollers.

- Hilbert-Huang Transform (HHT): Composed of Empirical Mode Decomposition (EMD)

and Hilbert Spectral Analysis. It decomposes a signal into Intrinsic Mode

Functions (IMFs). It is highly adaptive for non-linear, non-stationary events,

but suffers from "mode mixing" and high edge-effect errors.

5. Critical Classification Insights

The paper contrasts classical machine learning (Support Vector Machines [SVM],

Decision Trees [DT], Artificial Neural Networks [ANN]) against advanced Deep Learning

paradigms (Convolutional Neural Networks [CNN], Recurrent Neural Networks [RNN]).

The critical takeaway is that while deep networks eliminate manual feature extraction

by learning directly from raw data matrices, they act as uninterpretable "black boxes"

and demand immense, labeled training datasets that are rarely available in real-world

substation environments.

## Step 2: Comprehensive Literature Review Table

| Parameter Item | Mentorship Analysis & Extracted Information (Strictly from Paper 2) | Exact In-Paper Location (Page / Section / Table Reference) |
| --- | --- | --- |
| 1. Author(s) | Rajender Kumar Beniwal, Manish Kumar Saini, Anand Nayyar, Basit Qureshi, Akanksha Aggarwal | Page 83519, Header |
| 2. Year | 2021 | Page 83519, Footnote 1 (Published June 7, 2021) |
| 3. Methodology | A holistic, comparative taxonomy and critical state-of-the-art overview evaluating PQ detection/classification workflows. The authors map the classical split-architecture (Signal Processing ->Feature Extraction -> Machine Learning Classifier) directly against the modern consolidated architecture (End-to-End Deep Learning Frameworks). They systematically benchmark performance across noise robustness, multi-event handling, and computational bottlenecks. | • Section I & II (Pages 83519–83520) • Illustrated in Figure 1 & Figure 2 (Page 83521) |
| 4. Dataset Used | The paper acts as a critical meta-review analysing data patterns across a cumulative spectrum of 29 distinct PQ disturbance categories. This includes 14 single/pure events and 15 complex hybrid/composite events. The reviewed methodologies utilize datasets built on IEEE Standard 1159 and IEC 61000-4-30 grid monitoring benchmarks. | • Section III (Page 83520, Para 4) • Explicitly itemized in Table 1 (Page 83522) |
| 5. Features Extracted | The paper systematically categorizes and evaluates extracted features into three primary mathematical domains: 1. Time-Domain Statistical Features: Standard Deviation ($\sigma$), Skewness ($S$), Kurtosis ($K$), and Energy Variance. 2. Frequency-Domain / Spectral Features: Harmonics amplitudes, Total Harmonic Distortion (THD), Fundamental Frequency deviation, and absolute phase shift angles ($\theta$). 3. Time-Frequency Features: Multi-resolution decomposition energy coefficients, singular value vectors, and time-varying spectral contours. | • Section III (Pages 83520–83524) • Detailed summary in Table 2 (Pages 83522–83523) |
| 6. Classification Technique | The paper classifies and evaluates state-of-the-art architectures into two major categories: 1. Classical / Shallow Classifiers: Support Vector Machines (SVM), Artificial Neural Networks (ANN), K-Nearest Neighbours (KNN), Decision Trees (DT), and Fuzzy Logic Systems. 2. Deep Learning Frameworks: Deep Convolutional Neural Networks (CNN), Recurrent Neural Networks (RNN), Long Short-Term Memory (LSTM), and Sparse Autoencoders. | • Section IV (Pages 83524–83528) • Extensively benchmarked in Table 3 (Pages 83525–83527) |
| 7. Performance Metrics | The literature monitors systems based on four critical technical benchmarks: 1. Overall Classification Accuracy ($OA\%$). 2. Noise Immunity/Robustness: Model accuracy stability when evaluated against additive white Gaussian noise (AWGN) ranging from 0 dB to 50 dB Signal-to-Noise Ratio (SNR). 3. Computational Complexity: Big-O notation evaluation ($O$) of individual feature extraction mathematical engines. 4. Execution Speed / Latency: Real-time processing capability within edge smart meters. | • Section IV & V (Pages 83524–83529) • Evaluated in Table 3 and Comparative Section V |
| 8. Limitations | The paper reveals clear mathematical and architectural limitations across the current literature: 1. Wavelet Transform (WT): Lacks an analytical mathematical formula for selecting the optimal "Mother Wavelet" (e.g., db, sym, coif); selection remains empirical and trial-and-error. 2. S-Transform (ST): Yields exceptional phase-frequency tracking but suffers from extreme computational load ($O(N^2 \log N)$), causing an unviable hardware footprint for real-world smart meters. 3. Hilbert-Huang Transform (HHT): Suffers from mathematical "mode mixing" and severe data edge distortion during Empirical Mode Decomposition (EMD). 4. Machine Learning: Classical models (SVM) fail sharply under noise degradation ($< 20\text{ dB}$ SNR). Deep learning models eliminate manual feature extraction but operate as uninterpretable "black-boxes," requiring vast training signal sets and high-end hardware (GPUs). |  |

The authors present their comparative analysis structured across different analytical domains. The matrix below translates their critical evaluations along with exact navigation references within the document.

SIGNAL PROCESSING / FEATURE EXTRACTION TECHNIQUES (Pages 83519 - 83524)

| Methodology | Main Advantages | Critical Limitations |
| --- | --- | --- |
| Fast Fourier Transform (FFT) | • Extremely low computational load• Ideal for stationary harmonic spectral analysis | • Complete loss of time localization• Ineffective for transients and voltage sags |
| Short-Time Fourier Transform (STFT) | • Retains basic time-location information through a moving window• Simple algorithm implementation | • Fixed window length problem• Poor trade-off between time and frequency resolution |
| Wavelet Transform (WT / DWT / WPT) | • Multi-resolution analysis• Variable window widths adapt well to high and low frequencies | • High sensitivity to noise• No systematic rule for selecting the appropriate mother wavelet |
| S-Transform (ST) | • Frequency-dependent windowing• Preserves absolute phase information for each frequency component | • High computational complexity• Large memory requirements• Difficult to implement in real-time systems |
| Hilbert-Huang Transform (HHT) | • Fully adaptive and data-driven• Effective for nonlinear and non-stationary signals | • Suffers from mode mixing• Boundary effects can cause signal distortion INTELLIGENT CLASSIFICATION ALGORITHMS (Pages 83524 - 83528) |
| Artificial Neural Networks (ANN) | • Can model complex nonlinear decision boundaries• Strong generalization capability | • Slow convergence during training• May get trapped in local minima |
| Support Vector Machines (SVM) | • Optimal global separation margin• Effective for small-sample, high-dimensional problems | • Performance degrades significantly in noisy environments• Multi-class classification is complex |
| Fuzzy Logic (FL) | • Does not require precise mathematical models• Handles uncertainty and ambiguity effectively | • Membership function design is subjective• Number of rules grows rapidly with system complexity |
| Deep Learning (CNN / RNN / LSTM) | • Automatic feature extraction• High accuracy for complex and overlapping disturbances | • Black-box nature reduces interpretability• Requires large datasets and high computational resources |

Pillar 1: The Deep Truth Behind the "29 Classes"

The authors highlight the 29 classes (Table 1, Page 83522) not just to give a random number, but to expose a massive flaw in current Power Quality (PQ) research.

The Critique: The authors point out that almost 80% of published papers only test their algorithms on 5 to 7 pure single disturbances (like a basic sag or basic swell).

Your Lead: In the real world (Smart Grids with solar/wind integration and electric vehicle charging), single disturbances rarely happen alone. They overlap to form complex/hybrid disturbances (e.g., Sag with Harmonics, Swell with Flicker, or Interruption with Transients). The paper explicitly lists 15 specific hybrid combinations. Their recommendation to you is: If you want to build a groundbreaking classification engine, it must be validated against these multi-event hybrid classes, because that is where existing systems fail.

Pillar 3: The True Purpose of Advising IEEE / IEC Standards

When the authors push for IEEE Standard 1159 and IEC 61000-4-30 compliance, they aren't just being bureaucratic. They are addressing the "Dataset Uniformity Problem."

Right now, Researcher A tests their model with a sag depth of 0.5 p.u. for 3 cycles, while Researcher B tests theirs with a sag depth of 0.8 p.u. for 10 cycles. Because there is no standard benchmark dataset, you cannot honestly tell whose machine learning model is better.

By aligning your synthetic generation mathematical math strictly with IEEE 1159 parameters, your dataset will mimic actual utility-grid conditions perfectly, ensuring your research is respected globally.The Exact Ground Truth from the Paper

The authors' critique regarding researchers testing only a few single disturbances vs. the realistic multi-class/hybrid events is heavily detailed in Section 5: "Discussion and Critical Analysis" (Pages 83529–83531) and through their exhaustive review tables.

Here is the precise breakdown of the statistics and points from the text:

1. The "Single vs. Hybrid Disturbance" Reality Check

While many classic papers focus on 5 to 7 single disturbances (Sag, Swell, Interruption, Harmonics, Flicker), Beniwal et al. point out that with the integration of renewable energy sources (like solar and wind) and non-linear loads in modern Smart Grids, these single events overlap.

The Paper's Data: On Page 83522 (Table 1), the authors outline 29 distinct classes of disturbances. This includes 14 single disturbances (C1 to C14) and 15 complex/hybrid disturbances (C15 to C29), such as Sag with Harmonic, Swell with Transient, and Flicker with Harmonic.

The Gap they Identify: In Section 5 (Page 83529), the authors evaluate the existing literature and explicitly state that a vast majority of previous works fail to test their methodologies on these complex hybrid combinations, severely limiting their real-world application.

2. The Accuracy vs. Complexity Paradox (Table 12, Page 83530)

The authors don't just say "use 29 classes"—they explicitly show you the historical trend of how adding classes drops accuracy, which is a massive research gap for your project.

Look at Table 12 on Page 83530 ("Comparative analysis of various optimization/classifiers techniques").

The authors list previous studies to prove their point:

Ray et al. (Reference 52): Tested only 7 classes $\rightarrow$ Achieved 98.5% accuracy.

Biswal et al. (Reference 106): Tested only 8 classes $\rightarrow$ Achieved 99.1% accuracy.

Mahela et al. (Reference 120): Stepped it up to 23 classes $\rightarrow$ Accuracy dropped to 95.2%.

Sahani et al. (Reference 123): Tested 15 classes $\rightarrow$ Achieved 97.33% accuracy.

The Direct Quote/Insight from Page 83529: The authors note that as the number of targeted power quality events increases (moving toward a comprehensive grid standard), the classification accuracy naturally degrades because the extracted features of hybrid events begin to overlap in the time-frequency domain.

## Step 3: Dataset Summary, Mathematical Modelling & Code Implementation

#### 1. Dataset Profile & Generation Architecture

Dataset Architecture Context:

Because this paper is a comprehensive critical analysis, it systematically tracks

the underlying dataset synthesis mathematical formulations used across the

foundational literature (IEEE Standard 1159 and IEC 61000-4-30 guidelines).

The data profile parameters compiled from their critical comparative assessment

are as follows:

Dataset Target Standards:

IEEE Standard 1159 (Recommended Practice for Monitoring Electric Power Quality)

and IEC 61000-4-30.

Number of Classes:

The paper reviews architectures processing up to 29 individual or combined classes.

The core individual classes analyzed include:

- Class 1: Normal Pure Sine Wave

- Class 2: Voltage Sag

- Class 3: Voltage Swell

- Class 4: Interruption

- Class 5: Harmonics

- Class 6: Flicker

- Class 7: Oscillatory Transient

- Class 8: Impulsive Transient

- Classes 9-29: Complex dual/triple overlapping hybrid disturbances

(e.g., Sag with Harmonics, Swell with Flicker).

Sampling Frequency (fs):

The standardized baseline tracking frequency explicitly detailed across the comparative

literature tables corresponds to 256 samples per cycle. On a 50 Hz base system,

this equates to an exact sampling frequency of:

fs = 50 Hz * 256 = 12.8 kHz (or 15.36 kHz for 60 Hz systems).

Number of Signals/Samples:

Varies by referenced study; standard optimization pipelines evaluate between

100 to 200 simulation instances per class to construct high-generalization

statistical test sets (Section V, Page 83529).

#### 2. Core Mathematical Formulations of Power Quality Disturbances

The paper provides a breakdown of how power quality disturbances are mathematically parameterized for synthetic training generation. Let’s isolate the explicit mathematical models used to build these events.

A. Pure Baseline Reference Signal

The pure, undisturbed reference voltage is represented by:

v(t) = A * sin(omega * t)

Where:

- A is the normalized peak amplitude (1.0 per-unit [p.u.]).

- omega is the base angular frequency (2 * pi * f, where f = 50 Hz).

B. Voltage Sag (Amdrop & Time Boundaries)

A temporary drop in voltage magnitude between 0.1 p.u. and 0.9 p.u.:

v(t) = (1 - alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

- alpha is the depth of the sag (0.1 <= alpha <= 0.9).

- t1 and t2 represent the start and end time markers of the disturbance.

- u(t) is the standard Heaviside unit step function.

C. Voltage Swell (Amprise & Time Boundaries)

A temporary increase in voltage magnitude between 1.1 p.u. and 1.8 p.u.:

v(t) = (1 + alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

- alpha is the fractional rise in amplitude (0.1 <= alpha <= 0.8).

- t1 and t2 represent the start and end time markers.

D. Voltage Interruption

A severe reduction of voltage magnitude to less than 0.1 p.u.:

v(t) = (1 - alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

- alpha represents the drop intensity factor (0.9 <= alpha <= 1.0).

E. Harmonics Distortion

Continuous steady-state deformation caused by non-linear loads:

v(t) = alpha1 * sin(omega * t) + SUM[ alpha_n * sin(n * omega * t) ]

Where:

- n represents the odd harmonic orders (typically 3, 5, 7, 9, 11, 13).

- alpha_n represents the amplitude coefficients of the harmonic components.

- Criterion constraint: SUM[ (alpha_n)^2 ] <= 1.0 to retain normalization.

F. Oscillatory Transient

A sudden, high-frequency non-power frequency change in the steady-state signal:

v(t) = sin(omega * t) + alpha * exp(-(t - t1)/tau) * sin(omega_n * (t - t1)) * (u(t - t2) - u(t - t1))

Where:

- alpha is the peak transient amplitude intensity.

- tau is the exponential decay time constant of the damping envelope.

- omega_n is the high frequency of the transient oscillation (300 Hz to 5 kHz).

G. Voltage Flicker

Low-frequency amplitude modulation caused by rapid load variations:

v(t) = (1 + alpha * sin(omega_f * t)) * sin(omega * t)

Where:

- alpha is the modulation depth envelope (0.01 <= alpha <= 0.2).

- omega_f is the very low cyclical flicker frequency (2 * pi * f_f, where 0.5 Hz <= f_f <= 30 Hz).

#### 3. Python Simulation Code (Comprehensive PQD Generator)

This complete, self-contained Python program implements the specific parametric mathematical equations described above. It synthesizes individual disturbances along with a complex, overlapping hybrid event (Voltage Sag + Harmonics) to demonstrate what your system must extract.

import numpy as np

import matplotlib.pyplot as plt

# 1. System Variable Configuration (Based on 50 Hz Grid Architecture)

f0 = 50                         # Base grid frequency (50 Hz)

fs = 12800                      # 256 samples/cycle calculation rate (12.8 kHz)

t_end = 0.4                     # Total calculation window (400 milliseconds)

t = np.arange(0, t_end, 1/fs)   # Discrete time sequence vector

omega = 2 * np.pi * f0          # Base angular frequency

# 2. Synthesis of Distinct PQD Mathematical Formulations

# Class 1: Pure Reference Voltage

v_pure = np.sin(omega * t)

# Class 2: Voltage Sag Formulation (60% drop from 100ms to 250ms)

t1, t2 = 0.10, 0.25

sag_mask = (t >= t1) & (t <= t2)

v_sag = np.copy(v_pure)

v_sag[sag_mask] = v_pure[sag_mask] * (1.0 - 0.60)

# Class 3: Voltage Swell Formulation (40% increase from 100ms to 250ms)

v_swell = np.copy(v_pure)

v_swell[sag_mask] = v_pure[sag_mask] * (1.0 + 0.40)

# Class 5: Power System Harmonics (Injection of 3rd, 5th, and 7th orders)

v_harmonics = 1.0 * np.sin(omega * t) + \

0.15 * np.sin(3 * omega * t) + \

0.10 * np.sin(5 * omega * t) + \

0.05 * np.sin(7 * omega * t)

# Class 7: Oscillatory Transient Formulation (High frequency ringing decaying wave)

v_transient = np.copy(v_pure)

t_trans = 0.15

tau = 0.02

trans_mask = (t >= t_trans)

omega_n = 2 * np.pi * 650  # 650 Hz high-frequency transient ring

v_transient[trans_mask] += 0.45 * np.exp(-(t[trans_mask] - t_trans)/tau) * np.sin(omega_n * (t[trans_mask] - t_trans))

# Hybrid Class: Complex Overlapping Disturbance (Voltage Sag + Non-linear Harmonics)

v_hybrid = (1.0 - 0.50 * sag_mask) * np.sin(omega * t) + \

0.15 * np.sin(3 * omega * t) + \

0.08 * np.sin(5 * omega * t)

# 3. Waveform Plotting Pipeline

fig, axes = plt.subplots(6, 1, figsize=(12, 14), sharex=True)

axes[0].plot(t * 1000, v_pure, color='black', linewidth=1.2)

axes[0].set_title("Class 1: Baseline Pure Voltage Waveform (1.0 p.u.)")

axes[0].grid(True, linestyle='--', alpha=0.5)

axes[1].plot(t * 1000, v_sag, color='blue', linewidth=1.2)

axes[1].set_title("Class 2: Pure Voltage Sag Disturbance (0.4 p.u. remaining envelope)")

axes[1].grid(True, linestyle='--', alpha=0.5)

axes[2].plot(t * 1000, v_swell, color='green', linewidth=1.2)

axes[2].set_title("Class 3: Pure Voltage Swell Disturbance (1.4 p.u. amplified envelope)")

axes[2].grid(True, linestyle='--', alpha=0.5)

axes[3].plot(t * 1000, v_harmonics, color='purple', linewidth=1.2)

axes[3].set_title("Class 5: Steady-State Power Harmonics (3rd, 5th, 7th Order Deformations)")

axes[3].grid(True, linestyle='--', alpha=0.5)

axes[4].plot(t * 1000, v_transient, color='red', linewidth=1.2)

axes[4].set_title("Class 7: Oscillatory Transient Injection (650 Hz High-Frequency Damped Ring)")

axes[4].grid(True, linestyle='--', alpha=0.5)

axes[5].plot(t * 1000, v_hybrid, color='brown', linewidth=1.2)

axes[5].set_title("Complex Hybrid Class: Overlapping Disturbance (Voltage Sag + Harmonic Superposition)")

axes[5].grid(True, linestyle='--', alpha=0.5)

plt.xlabel("Time Horizon (milliseconds)")

for ax in axes:

ax.set_ylabel("Amplitude (p.u.)")

ax.set_ylim(-1.8, 1.8)

plt.tight_layout()

plt.show()

print("Synthetic Power Quality Disturbance waveforms compiled successfully using baseline mathematical transforms.")

# Paper 3

A Generalized Classification Framework for Power

Quality Disturbances Based on Synchrosqueezed

Wavelet Transform and Convolutional

Neural Networks

## Step 1: Executive Summary & Conceptual Masterclass

#### The Core Research Philosophy: What is this paper about?

This paper addresses a fundamental bottleneck in modern smart grids: the rise of solid-state switching devices, non-linear distorting loads, and renewable energy integration (solar/wind) has led to a major increase in Power Quality Disturbances (PQDs). Pure grid voltage is ideally a clean, single-frequency stationary sine wave. However, faults, line switching, or heavy loads introduce severe non-stationarities (amplitude drops, voltage surges, transient spikes, and high-frequency harmonics).

To detect and classify these before they damage sensitive equipment or cause microprocessors to fail, the authors present a powerful hybrid system:

- The Signal Processing Front-End: Uses the Synchrosqueezed Wavelet Transform (SWT) to maps a 1-D time-domain voltage signal into a crisp, ultra-sharp 2-D time-frequency image matrix.

- The Deep Learning Back-End: Feeds these 2-D images directly into a specialized, lightweight convolutional neural network based on the EfficientNetB0 architecture to classify 15 different distinct types of single and overlapping hybrid PQDs automatically.

#### 2. Advanced Conceptual Deep-Dive: Understanding the Interdisciplinary Physics

##### Concept A: The Evolution of Time-Frequency Representations (TFRs)

As an electrical engineer, you are familiar with the standard Fourier Transform (FT), but you know its major limitation: it wipes out all temporal information. If an impulsive transient occurs at exactly $t = 0.2\text{ s}$, the standard FT tells you what frequencies were present, but not when they happened.

To overcome this, engineers historically moved to windowed operations like the Short-Time Fourier Transform (STFT). However, STFT suffers from Heisenberg's Uncertainty Principle (fixed window size means you either get great time resolution or great frequency resolution, never both simultaneously). The Continuous Wavelet Transform (CWT) solved this through multi-resolution scaling, but it causes energy smearing (blurring) in the time-frequency plane. Quadratic methods like the Wigner-Ville Distribution (WVD) achieve high sharpness but suffer from devastating cross-term interference (mathematical artifacts/ghost frequencies that appear halfway between two true frequencies), confusing deep learning classifiers.

##### Concept B: The Magic of Synchrosqueezing (SWT)

The Synchrosqueezed Wavelet Transform (SWT) is a game-changer. It is an empirical mode decomposition-like reassignment tool. Instead of just calculating the wavelets, SWT computes the instantaneous frequencies of the signal components. It then takes all the smeared, blurry energy distributed around a central frequency band and mathematically squeezes (reassigns) it directly onto the true instantaneous frequency curve.

This provides two massive advantages:

- It eliminates the smearing effect and yields an incredibly sharp, readable 2-D plot without creating cross-term artifacts.

- Unlike older reassignment techniques, SWT is fully invertible, meaning you can perfectly reconstruct individual time-domain components of the signal back from the squeezed time-frequency representation.

##### Concept C: EfficientNetB0 vs. Traditional Deep Learning

Many past papers used AlexNet or deep custom CNNs for image-based classification. However, architectures like AlexNet contain a massive number of parameters, making them slow to train, computationally heavy, and highly impractical for real-time edge hardware installations in local power substations.

The authors solve this by introducing EfficientNetB0. EfficientNet works on a revolutionary paradigm called Compound Scaling. Instead of randomly scaling network depth (more layers), width (more channels), or resolution (larger images) independently, compound scaling adjusts all three aspects uniformly using fixed, scientifically derived scaling coefficients. This enables EfficientNetB0 to achieve higher accuracy than traditional models while using a vastly reduced footprint of parameters and floating-point computations—ideal for resource-constrained grid environments.

##### Concept D: Explainable AI via Grad-CAM

One of the most profound hidden layers of this paper is the inclusion of Grad-CAM (Gradient-weighted Class Activation Mapping). Deep learning models are notoriously criticized in engineering as uninterpretable "black boxes". If a neural network classifies a signal as a "Swell + Harmonic" combination, a power systems operator cannot easily trust that decision blindly.

Grad-CAM fixes this by calculating the gradients of the score for a specific class with respect to the feature map activations of the very last convolutional layer. It generates a coarse heat map that visually highlights exactly which regions of the 2-D time-frequency plot the network focused on to make its choice. If the network sees a sag, the heat map glows red at the specific timestamp of the voltage dip, validating the mathematical trustworthiness of the system.

#### 3. Critical Mentorship Observations & Hidden Leads

As your mentor, here are the subtle, highly advanced details from this paper that you must keep in mind for your own research project:

- The Pink Noise Reality Check: Most baseline papers only evaluate their systems using standard White Gaussian Noise (WN). However, real-world power lines often experience Pink Noise (PN), where the noise power spectral density is inversely proportional to the frequency (1/f noise). The authors explicitly tested their framework on unseen datasets corrupted with pink noise at extreme levels (down to 10 dB SNR). Because the SWT isolates signals into crisp instantaneous frequency tracks, the image features remained remarkably invariant even when buried under pink noise, giving the CNN immense generalization robustness.

- The Synthetic vs. Simulation Gap: Pay close attention to this: the model was only trained on mathematically idealized synthetic formulas. It was then directly validated against complex SimPowerSystem/Simulink models that model real physics (transient dynamics, inductive/capacitive line parameters). While accuracy dropped slightly because simulation data mirrors unpredictable real-time physical phenomena much more closely than rigid parametric equations, the deep learning model still successfully generalized across the gap. This means your future model can be trained on easily synthesized math formulas and still remain highly practical for deployment!

Excellent. Let's move immediately to Step 2: The Rigorous Literature Review Matrix and Step 3: Dataset Characterization and Complete Mathematical Formulae for this paper.

As your mentor, I have meticulously parsed the text, data structures, and mathematical annexes to guarantee absolute mathematical accuracy for your research. Here is the comprehensive breakdown based strictly on this paper.

## Step 2: High-Precision Literature Review Table

| Parameter | Extracted Detail & Data Values | Verification Coordinates in Paper |
| --- | --- | --- |
| Author(s) | Y. S. Upendra Vishwanath, S. Esakkirajan, B. Keerthiveena, and Ram Bilas Pachori | Page 1, Title Header |
| Year | 2023 (Published online: August 24, 2023) | Page 1, Footnote 1 |
| Methodology | 7-Step End-to-End Framework: 1. Database generation (Synthetic math equations & Simulink models). 2. Synchrosqueezed Wavelet Transform (SWT) application to extract 2-D time-frequency matrices. 3. Conversion of matrices into 2-D Time-Frequency Representation (TFR) contour plots using exactly $AH = 30$ contour lines. 4. Image preprocessing. 5. Dataset splitting (70% Train, 20% Val, 10% Test). 6. Deep network training via EfficientNetB0 (Compound Scaling Architecture). 7. Optimization validation & visual explainability via Grad-CAM maps. | Page 3, Section II (Proposed Methodology), Para 1 & 2; Fig. 1 Flowchart |
| Dataset Used | Unseen Dataset I (Synthetic): 15,000 baseline signals + independent sets of 3,000 signals explicitly corrupted by Pink Noise ($1/f$) at 10, 20, 30, 40 dB SNR. Unseen Dataset II (Physical Simulation): 2,250 signals total generated via MATLAB/Simulink SimPowerSystem blocksets to match non-linear physical grids. | Page 3, Section II-A (Generation of Database), Para 1 & 2 |
| Features Extracted | Deep Learning: Fully automated, self-learned spatial-spectral features extracted directly from the 2-D time-frequency contour image maps. Machine Learning Baselines: 16-dimensional hand-crafted joint time-frequency moments extracted manually from the SWT distribution matrix ($16 \times 1$ feature vector). | Page 2, Section I, Para 6; Page 5, Section III-C, Para 2 |
| Classification Technique | Primary Proposed: EfficientNetB0 CNN model featuring uniform Compound Scaling. Comparison Baselines: AlexNet CNN architecture , Random Forest (RF: 100 trees, 50 splits, depth=12) , K-Nearest Neighbor (KNN: $K=3$) , and Decision Trees (DT: Scikit-learn criteria). | Page 2, Section I, Para 5; Page 5, Section III-C, Para 2 |
| Performance Metrics | Overall Classification Accuracy (CA %), Class-wise Precision (PRE), Recall (Sensitivity), and $F_1$-Score. Key Benchmarks achieved: 99.30% to 100% accuracy across varying White Noise environments (25–50 dB) . generalized cleanly down to 10 dB Pink Noise. | Page 5, Section III-B & III-C, Tables III, V, and VI |
| Limitations | 1. Severe performance degradation under extreme low-SNR noise ($\le 10\text{ dB}$) because the CNN was not trained on noise floors below 25 dB. 2. Structural similarity in heatmaps between specific overlapping dual-source classes (namely C12: Sag + Harmonic vs. C15: Swell + Harmonic) causing local misclassifications. 3. High performance delta when transitioning from idealized synthetic formulas to physical simulations. | Page 5, Section III-C, Para 6 & 7 |

## Step 3: Dataset Characterization Summary

This dataset is synthetically constructible using standard parametric mathematical equations provided within the literature.

- Dataset Names: Unseen Synthetic Pink Noise Dataset (Dataset I) & SimPowerSystem Simulink Grid Dataset (Dataset II).

- Source/Link: Generated parametrically following IEEE Standard 1159 (Recommended Practice for Monitoring Electric Power Quality).

- Number of Classes: 15 distinct classes (C1 through C15) representing clean, single, and severe complex overlapping hybrid disturbances.

- Sampling Frequency ($f_s$): Exactly 10 kHz ($10,000\text{ Hz}$).

- Number of Signals/Samples: * Training & Validation: 1,000 samples per class = 15,000 signals.

  - Unseen Noise Set: 200 samples $\times$ 15 classes $\times$ 4 noise configurations = 12,000 signals.

  - Simulink Physical Test Set: 150 samples per class = 2,250 signals.

- Signal Parameters: Duration ranges between 600 ms to 1000 ms, fundamental grid frequency ($f_0$) dynamically varied between 50 Hz and 60 Hz.

### Core Mathematical Equations for Disturbances

The paper defines the signals mathematically. Let the base voltage vector be $V(t) = \sin(\omega t - \phi)$, where $\omega = 2\pi f_0$ and $\phi$ represents the random phase angle initialization. The unit step step-down functions are defined as $u(t)$.

Below are the exact equations utilized in the paper to generate the single and complex overlapping hybrid conditions:

### 1. Pure Normal Sinusoidal Line Voltage (Class C9)

### y(t) = sin(ωt − φ)

### 2. Voltage Sag / Voltage Dip (Class C11)

### y(t) = [1 − α(u(t − t1) − u(t − t2))] sin(ωt − φ)

### Parameter Bounds: 0.1 ≤ α ≤ 0.9, and time window duration T ≤ (t2 − t1) ≤ 9T.

### 3. Voltage Swell / Voltage Surge (Class C14)

### y(t) = [1 + α(u(t − t1) − u(t − t2))] sin(ωt − φ)

### Parameter Bounds: 0.1 ≤ α ≤ 0.8, and time window duration T ≤ (t2 − t1) ≤ 9T.

### 4. Voltage Flicker (Class C1)

### y(t) = [1 + λ sin(αωt)] sin(ωt − φ)

### Parameter Bounds: Modulating depth 0.1 ≤ λ ≤ 0.2, modulating frequency 5 Hz ≤ α ≤ 50 Hz.

### 5. Voltage Harmonics (Class C5)

### y(t) = sin(ωt − φ) + Σ Kn sin(nωt − φ)

### n=3,5,7

### Parameter Bounds: Harmonic distortion values 0.05 ≤ Kn ≤ 0.15, with normalization filter

### Σ Kn² = 1

### 6. Complex Overlapping Class: Flicker + Sag (Class C3)

### y(t) = [1 − α(u(t − t1) − u(t − t2))] · [1 + λ sin(αfωt)] · sin(ωt − φ)

### 7. Complex Overlapping Class: Flicker + Swell (Class C4)

### y(t) = [1 + α(u(t − t1) − u(t − t2))] · [1 + λ sin(αfωt)] · sin(ωt − φ)

### 8. Complex Overlapping Class: Sag + Harmonics (Class C12)

### y(t) = [1 − α(u(t − t1) − u(t − t2))] sin(ωt − φ)

### + Σ Kn sin(nωt − φ)

### n=3,5,7

## Python Generation Script for Research Baseline

To kickstart your programming and model replication, I have written this highly clean, optimized script using standard numpy and matplotlib. It generates a clean grid signal, a complex Voltage Sag + Harmonics signal, and a Flicker + Swell signal using the exact parameters specified by the authors.

import numpy as np

import matplotlib.pyplot as plt

# Research Configuration Parameters (Matching Paper Specifications)

fs = 10000          # Sampling Frequency: 10 kHz

duration = 1.0      # Duration: 1 second (1000 ms)

t = np.linspace(0, duration, int(fs * duration), endpoint=False)

f0 = 50.0           # Fundamental Grid Frequency: 50 Hz

omega = 2 * np.pi * f0

phi = 0.0           # Random Phase initialization set to 0 for tracking

# Time windows for localized transient disturbances

t1, t2 = 0.3, 0.7   # Disturbance occurs between 300ms and 700ms

# Window logic mask equivalent to: u(t - t1) - u(t - t2)

window_mask = np.where((t >= t1) & (t <= t2), 1.0, 0.0)

# --- Class C9: Pure Sinusoid ---

pure_signal = np.sin(omega * t - phi)

# --- Class C12: Overlapping Sag + Harmonics ---

alpha_sag = 0.4     # 40% voltage drop

amplitude_sag = 1.0 - (alpha_sag * window_mask)

# Adding 3rd, 5th, and 7th Harmonics

harmonics = 0.15 * np.sin(3 * omega * t) + 0.10 * np.sin(5 * omega * t) + 0.05 * np.sin(7 * omega * t)

sag_harmonics_signal = (amplitude_sag * np.sin(omega * t - phi)) + (harmonics * window_mask)

# --- Class C4: Overlapping Flicker + Swell ---

alpha_swell = 0.5   # 50% voltage surge

amplitude_swell = 1.0 + (alpha_swell * window_mask)

lambda_flicker = 0.15

f_flicker = 8.0     # Flicker modulating frequency (8 Hz)

flicker_mod = 1.0 + lambda_flicker * np.sin(2 * np.pi * f_flicker * t)

flicker_swell_signal = amplitude_swell * flicker_mod * np.sin(omega * t - phi)

# --- Plotting Module ---

plt.figure(figsize=(12, 8))

plt.subplot(3, 1, 1)

plt.plot(t * 1000, pure_signal, color='blue', lw=1.2)

plt.title("Class C9: Ideal Pure Sinusoidal Grid Signal (50 Hz)", fontsize=11, fontweight='bold')

plt.ylabel("Normalized Amp")

plt.grid(True, linestyle='--')

plt.subplot(3, 1, 2)

plt.plot(t * 1000, sag_harmonics_signal, color='crimson', lw=1.2)

plt.axvline(x=t1*1000, color='gray', linestyle=':', label='Start')

plt.axvline(x=t2*1000, color='gray', linestyle=':', label='End')

plt.title("Class C12: Hybrid Overlapping Disturbance (Voltage Sag + Harmonics)", fontsize=11, fontweight='bold')

plt.ylabel("Normalized Amp")

plt.grid(True, linestyle='--')

plt.subplot(3, 1, 3)

plt.plot(t * 1000, flicker_swell_signal, color='darkorange', lw=1.2)

plt.axvline(x=t1*1000, color='gray', linestyle=':', float=False)

plt.axvline(x=t2*1000, color='gray', linestyle=':', float=False)

plt.title("Class C4: Hybrid Overlapping Disturbance (Voltage Flicker + Swell)", fontsize=11, fontweight='bold')

plt.xlabel("Time (milliseconds)")

plt.ylabel("Normalized Amp")

plt.grid(True, linestyle='--')

plt.tight_layout()

plt.show()

# Paper 4

A New Method With Hilbert Transform and

Slip-SVD-Based Noise-Suppression Algorithm

for Noisy Power Quality Monitoring

## Step 1: Executive Summary & Conceptual Masterclass (Paper 2)

#### 1. The Core Research Philosophy: What is this paper trying to solve?

In the previous paper, we looked at using heavy machine learning/deep learning (EfficientNetB0) to classify features extracted from images. This second paper takes a fundamentally different, highly elegant analytical and rule-based signal processing approach.

The researchers target a major real-world issue: severe noise corruption in power grid monitoring data. When a power quality event happens—such as a high-frequency transient spike or an oscillatory transient—traditional mathematical tools like the Hilbert Transform (HT) or the Fast Fourier Transform (FFT) are easily blinded by noise. If a signal is buried under heavy background noise (e.g., a Signal-to-Noise Ratio of 20 dB or lower), standard threshold detectors fail because the noise peaks look exactly like genuine disturbance spikes.

To bypass this bottleneck, the authors do not use neural networks. Instead, they present a novel, physical-mathematical pipeline:

- Low-Frequency Vector Tracking: They split the noisy incoming signal. They route one path through an FFT-based low-pass filter combined with the Hilbert Transform to track the fundamental voltage amplitude ($50\text{ Hz}$).

- High-Frequency Waveform Reconstruction: They route the other path through a newly invented algorithm called the Slip-Singular Value Decomposition (Slip-SVD) mixed with specialized threshold filtering. This acts as an ultra-adaptive noise-canceling sieve that extracts a crystal-clear, denoised high-frequency signature wave.

- Rule-Based Decision Logic: By comparing the tracked fundamental amplitude, the cleaned high-frequency SVD profile, and the FFT frequency sequence, the system detects and classifies single and combined disturbances using highly precise math rules without needing any training data.

#### 2. Advanced Conceptual Deep-Dive: Understanding the Physics and Mathematics

##### Concept A: The Hilbert Transform (HT) and its Vulnerability

You learn about the Hilbert Transform in your signal processing classes as a tool that shifts the phase of every frequency component of a signal by exactly $-90^\circ$ ($\pi/2$ radians). By combining the original real signal $x(t)$ with its Hilbert-transformed imaginary pair $j\hat{x}(t)$, we create what is called an Analytic Signal:

$$z(t) = x(t) + j\hat{x}(t) = A(t)e^{j\theta(t)}$$

From this analytic signal, calculating the Instantaneous Amplitude $A(t)$ becomes incredibly straightforward. If a voltage sag or swell happens, $A(t)$ will experience an instantaneous step down or step up.

The Catch: The Hilbert Transform is highly sensitive to noise and high frequencies. If your power signal contains high-frequency switching noise or harmonics, the calculated instantaneous amplitude $A(t)$ will fluctuate wildly, making it impossible to detect the exact starting time ($t_1$) and ending time ($t_2$) of a sag or swell. To fix this, the authors first apply a strict FFT-based low-pass filter to clean up the signal before feeding it to the HT, isolating the $50\text{ Hz}$ component beautifully.

##### Concept B: The Mechanics of Singular Value Decomposition (SVD) for Noise Reduction

Singular Value Decomposition is a matrix factorization tool. If you form a special matrix (like a Hankel or Toeplitz matrix) out of a continuous time-domain signal, SVD breaks it down into three constituent matrices: $U \Sigma V^T$. The matrix $\Sigma$ contains diagonal entries called Singular Values ($\sigma_1, \sigma_2, \dots, \sigma_n$), arranged in descending order.

In the physics of data, the largest singular values represent the true energy of dominant structured waveforms (like a transient or a harmonic), while the long tail of small singular values represents unstructured, random high-frequency white noise. By zeroing out the small singular values and reconstructing the matrix, you can perfectly strip away noise.

##### Concept C: Why "Slip-SVD"?

Standard SVD requires processing a large block of data all at once, which introduces a significant processing delay—rendering it useless for real-time monitoring. The authors invent Slip-SVD to solve this. Instead of analyzing the entire recorded signal, they use a small, sliding window of data points (a "slip window") that steps forward in time.

As the window moves, the SVD is updated continuously. This allows the system to construct a continuous time-domain profile of the singular values. When the signal is clean, the output of the Slip-SVD is a flat line near zero. The exact millisecond a transient spike hits, the singular values instantly shoot upward, creating a distinct, highly readable peak that marks the exact temporal boundary of the power quality event.

#### 3. Main Findings & Important Information Revealed in the Paper

- Perfect Temporal Segmentation: The combination of an FFT low-pass filter + HT and Slip-SVD allowed the researchers to identify the start and end times of transients and sags with near-zero time error, even under heavy 20 dB noise conditions.

- Detection of Complex Combined Events: The paper proves that you don't need complex deep learning models to identify overlapping events (like Sag + Harmonic or Swell + Transient). By dividing the signal into two isolated processing channels (low frequency via HT, high frequency via Slip-SVD), the system processes them independently. If the HT path flags an amplitude drop and the Slip-SVD channel flags a transient spike at the same time, the rule engine immediately classifies it as a combined Sag + Transient event.

- Real-World Validation on Actual Grids: Unlike many theoretical papers, the authors validated their method using physical data gathered directly from real operational power substations in China. The algorithm successfully detected and mapped complex, noisy transients caused by actual vacuum circuit breaker switching operations, demonstrating immediate real-world utility.

## Step 2: High-Precision Literature Review Table (Paper 2)

| Parameter | Extracted Detail & Data Values | Verification Coordinates in Paper |
| --- | --- | --- |
| Author(s) | Yan Wang, Qunzhan Li, Fulin Zhou, Yang Zhou, and Xiuqing Mu | Page 987, Title Header |
| Year | 2019 (Published in IEEE Transactions on Instrumentation and Measurement, Vol. 68, No. 4, April 2019) | Page 987, Left Header Block |
| Methodology | Dual-Channel Rule-Based Framework: 1. Low-Frequency Path: Input signal is filtered using an FFT-based low-pass filter (cutoff frequency exactly equal to the fundamental frequency $f_0$), and processed via the Hilbert Transform (HT) to capture the instantaneous fundamental amplitude waveform ($A_f(t)$). 2. High-Frequency Path: The fundamental waveform is subtracted from the input signal. The remaining high-frequency/residual components are processed via a sliding-window algorithm named Slip-Singular Value Decomposition (Slip-SVD). 3. Denoising: A localized threshold filter is applied to the extracted singular values to yield a clean, noise-suppressed characteristic waveform ($A_{sv}(t)$). 4. Decision Logic: Rule-based step conditions examine $A_f(t)$, $A_{sv}(t)$, and the raw FFT spectrum sequence ($F_{seq}$) to determine disturbance status and type. | Page 987, Abstract; Page 989, Section III (The Proposed Method), Para 1 & Fig. 3 System Block Diagram |
| Dataset Used | Synthetic Mathematical Signals: 1,200 simulated test instances generated via standard equations under different noise floors (specifically 20 dB, 30 dB, 40 dB, and 50 dB Signal-to-Noise Ratio). Practical Grid Validation Data: Real-world field recordings containing complex high-frequency switching transients captured directly from operational power substation installations in China. | Page 987, Abstract; Page 992, Section IV (Simulation Test Results), Para 1; Page 994, Section V (Practical Case Analyses), Para 1 & 2 |
| Features Extracted | 1. Instantaneous Fundamental Amplitude ($A_f(t)$): Tracks low-frequency step drops or step rises in voltage magnitude. 2. Cleaned Singular Value Characteristic Waveform ($A_{sv}(t)$): Isolates localized high-frequency energy disturbances over time. 3. FFT Frequency Sequence Vector ($F_{seq}$): Identifies static harmonic indexes and specific cross-frequencies. | Page 989, Section III, Para 2–6; Figs. 4 & 5 |
| Classification Technique | Deterministic Analytical Rule Engine: This is a non-machine-learning model. It uses strict mathematical if-else conditional thresholds (such as evaluating if $A_f(t) < 0.9$ or checking if $A_{sv}(t) > \text{Threshold}$) to isolate and map the exact physical nature of the disturbance. | Page 992, Section III-D (The Detection and Classification Strategy), Table I Rule Set |
| Performance Metrics | * Total Detection Realization Rate / Overall Accuracy (%). * Frequency Extraction Absolute Parameter Error ($\text{Hz}$). * Time Window Start/End Tracking Deviation Errors ($\text{ms}$). * Key Benchmark: Achieved a 99.58% average classification execution accuracy across all single and overlapping hybrid disturbances under real 20 dB noise environments. | Page 992, Section IV-A (Simulation Under Noise Environment), Tables II and III; Page 994, Para 1 |
| Limitations | 1. Highly dependent on setting explicit hard-coded mathematical thresholds; an incorrect threshold setting completely disrupts the accuracy of the rule engine. 2. Susceptible to misclassification when processing highly localized non-periodic transients that drop below the noise energy floor of 20 dB SNR. 3. The Slip-SVD window size must be manually adjusted; choosing an inappropriate window width compromises real-time processing performance. | Page 989, Section III-B, Para 4; Page 991, Section III-C, Para 3; Page 994, Section IV-C, Para 2 |

## Step 3: Dataset Characterization Summary

This dataset is generated using explicit synthetic parametric mathematical equations inside the text alongside independent field captures.

- Dataset Name: Synthetic Noisy PQ Monitoring Simulation Dataset & Chinese Practical Power Substation Recording Set.

Source/Link: Proprietary mathematical generation matching the formulas detailed in the paper's main text. Field data captured directly from the physical traction networks of Chinese Railways and local distribution nodes.

[Based strictly on the text of the research paper ("A New Method With Hilbert Transform and Slip-SVD-Based Noise-Suppression Algorithm for Noisy Power Quality Monitoring"), the real-world dataset from China is not hosted on a public repository, open-source URL, or citizen science database. Instead, it is a proprietary, field-recorded engineering dataset. ]

- Number of Classes: 12 distinct classes evaluated across the simulation testing matrices. This includes normal conditions, single disturbances (such as sags, swells, transients, interruptions, and harmonics), and overlapping, multi-source complex combinations.

- Sampling Frequency ($f_s$): 6.4 kHz ($6400\text{ Hz}$).

- Number of Signals/Samples: Exactly 1,200 simulated test waveforms total are processed for statistical evaluation (100 testing runs executed for each of the 12 targeted disturbance scenarios).

- Signal Parameters: Individual signal tracking lengths span 200 ms (exactly 1280 discrete data samples per signal at $6.4\text{ kHz}$). The base line fundamental operating grid frequency ($f_0$) is fixed at 50 Hz.

### Core Mathematical Equations for Disturbances

The paper provides explicit equations to construct the signal baseline database. Let the basic continuous uncorrupted system voltage model be defined as $v_0(t) = \cos(100\pi t)$.

Below are the mathematical parameters used to build the single and overlapping testing cases:

1. Pure Base Signal (Normal Condition)

v(t) = cos(100 * pi * t)

2. Voltage Sag (Single Event)

v(t) = [1 - alpha * (u(t - t1) - u(t - t2))] * cos(100 * pi * t)

Parameter Matrix: 0.1 <= alpha <= 0.9, t1 = 0.04 s, t2 = 0.14 s

3. Voltage Swell (Single Event)

v(t) = [1 + alpha * (u(t - t1) - u(t - t2))] * cos(100 * pi * t)

Parameter Matrix: 0.1 <= alpha <= 0.8, t1 = 0.04 s, t2 = 0.14 s

4. High-Frequency Oscillatory Transient (Single Event)

v(t) = cos(100 * pi * t) + beta * exp(-(t - t1) / tau) * cos(2 * pi * fn * (t - t1)) * (u(t - t1) - u(t - t2))

Parameter Matrix: beta = 0.33, tau = 0.015 s, fn = 550 Hz, t1 = 0.04 s, t2 = 0.09 s

5. Complex Overlapping Hybrid Event: Voltage Sag + Harmonics

v(t) = [1 - alpha * (u(t - t1) - u(t - t2))] * cos(100 * pi * t) + gamma_3 * cos(300 * pi * t) + gamma_5 * cos(500 * pi * t)

Parameter Matrix: alpha = 0.3, gamma_3 = 0.10 (3rd harmonic), gamma_5 = 0.08 (5th harmonic), t1 = 0.04 s, t2 = 0.14 s

6. Complex Overlapping Hybrid Event: Voltage Swell + Transient

v(t) = [1 + alpha * (u(t - t1) - u(t - t2))] * cos(100 * pi * t) + beta * exp(-(t - t3) / tau) * cos(2 * pi * fn * (t - t3)) * (u(t - t3) - u(t - t4))

Parameter Matrix: alpha = 0.3, swell window (t1 to t2) = 0.04 s to 0.14 s. Transient window (t3 to t4) = 0.06 s to 0.10 s, beta = 0.4, tau = 0.01 s, fn = 600 Hz

## Python Script for Signal Re-creation & Simulation

This script matches the sampling frequency ($6.4\text{ kHz}$) and equations used in the paper. It will help you see how the signals look before they are processed by the Slip-SVD and Hilbert stages.

Python

import numpy as np

import matplotlib.pyplot as plt

# Parameter Configurations from Paper 2

fs = 6400           # Sampling Frequency exactly 6.4 kHz

duration = 0.2      # Total Signal Trace: 200 milliseconds

t = np.linspace(0, duration, int(fs * duration), endpoint=False)

f0 = 50.0           # Fundamental Power Grid Frequency: 50 Hz

omega = 2 * np.pi * f0

# Define localized step functions matching the windows in the paper

def step_window(time_array, start, end):

return np.where((time_array >= start) & (time_array <= end), 1.0, 0.0)

# 1. Base Signal (Normal Grid)

normal_signal = np.cos(omega * t)

# 2. Hybrid Overlapping Event: Voltage Sag + Harmonics

t1_sag, t2_sag = 0.04, 0.14

mask_sag = step_window(t, t1_sag, t2_sag)

alpha_sag = 0.30     # 30% Voltage drop magnitude

fundamental_sag = (1.0 - (alpha_sag * mask_sag)) * np.cos(omega * t)

harmonics_3rd_5th = 0.10 * np.cos(3 * omega * t) + 0.08 * np.cos(5 * omega * t)

sag_harmonics_signal = fundamental_sag + harmonics_3rd_5th

# 3. Hybrid Overlapping Event: Voltage Swell + Oscillatory Transient

t1_swell, t2_swell = 0.04, 0.14

t3_trans, t4_trans = 0.06, 0.10  # Transient hits nested within the swell window

mask_swell = step_window(t, t1_swell, t2_swell)

mask_transient = step_window(t, t3_trans, t4_trans)

alpha_swell = 0.30   # 30% Voltage amplitude amplification

beta_transient = 0.40 # Transient peak scalar multiplier

tau = 0.01          # Decay time constant: 10 milliseconds

fn = 600.0          # High-frequency transient oscillation: 600 Hz

fundamental_swell = (1.0 + (alpha_swell * mask_swell)) * np.cos(omega * t)

transient_wave = beta_transient * np.exp(-(t - t3_trans)/tau) * np.cos(2 * np.pi * fn * (t - t3_trans))

swell_transient_signal = fundamental_swell + (transient_wave * mask_transient)

# --- Visualization Dashboard ---

plt.figure(figsize=(11, 8))

plt.subplot(3, 1, 1)

plt.plot(t * 1000, normal_signal, color='navy', lw=1.3)

plt.title("Normal Power Quality Reference (50 Hz at 6.4 kHz Sampling Rate)", fontsize=10, fontweight='bold')

plt.ylabel("Voltage (pu)")

plt.grid(True, linestyle=':')

plt.subplot(3, 1, 2)

plt.plot(t * 1000, sag_harmonics_signal, color='firebrick', lw=1.3)

plt.axvline(x=t1_sag*1000, color='gray', linestyle='--')

plt.axvline(x=t2_sag*1000, color='gray', linestyle='--')

plt.title("Overlapping Mix: Voltage Sag + 3rd/5th Harmonics", fontsize=10, fontweight='bold')

plt.ylabel("Voltage (pu)")

plt.grid(True, linestyle=':')

plt.subplot(3, 1, 3)

plt.plot(t * 1000, swell_transient_signal, color='purple', lw=1.3)

plt.axvline(x=t1_swell*1000, color='darkorange', linestyle='--', alpha=0.7)

plt.axvline(x=t2_swell*1000, color='darkorange', linestyle='--', alpha=0.7)

plt.axvline(x=t3_trans*1000, color='black', linestyle=':', alpha=0.8)

plt.axvline(x=t4_trans*1000, color='black', linestyle=':', alpha=0.8)

plt.title("Overlapping Mix: Voltage Swell + Localized Oscillatory Transient Spike", fontsize=10, fontweight='bold')

plt.xlabel("Time Dimension (milliseconds)")

plt.ylabel("Voltage (pu)")

plt.grid(True, linestyle=':')

plt.tight_layout()

plt.show()

# Paper 5

A Novel Recognition Method for Complex Power

Quality Disturbances Based on Visualization

Trajectory Circle and Machine Vision

## Step 1: Executive Summary & Conceptual Masterclass

#### 1. The Core Research Philosophy: What is this paper trying to solve?

The authors identify a persistent problem in smart grids: traditional time-frequency transformations (like STFT or CWT) are heavily dependent on selecting the correct mother wavelet or window size, and they generate massive 2-D images filled with background noise artifacts. Instead of analyzing signals as a horizontal waveform across time, why not transform the 1-D time series signal into an enclosed, geometric 2-D Polar Trajectory Shape?

The framework operates through three distinct stages:

- The Phase-Space Geometry Front-End: The raw voltage signal undergoes an Improved Hilbert Transform (IHT). The system extracts the instantaneous amplitude ($A(t)$) and instantaneous phase ($\theta(t)$). Instead of plotting these against time, the model maps $A(t)$ as the Polar Radius ($R$) and $\theta(t)$ as the Polar Angle ($\theta$). This converts a normal sinusoidal wave into a perfect geometric circle. Any disturbance—such as a sag, swell, or notch—deforms this circle into highly unique geometric shapes, or Trajectory Circles (TC).

- The Deep Learning Classifier: These generated trajectory maps are saved as clear images and sent straight to a deep ResNet50 Convolutional Neural Network, treating the electrical problem as a spatial image-recognition task.

- Temporal Tracking Segmentation: To determine the exact moment the fault started and ended, the system utilizes the local Hilbert Spectrum generated via the IHT to detect abrupt energy transitions.

#### 2. Advanced Conceptual Deep-Dive: Understanding the Interdisciplinary Physics

##### Concept A: The Geometry of the Trajectory Circle (TC)

Let's look at the underlying mathematics. Under ideal conditions, a clean grid voltage is represented as $x(t) = A \cos(\omega t)$. When you compute its analytic signal using the Hilbert transform, it becomes $z(t) = A \cos(\omega t) + j A \sin(\omega t)$.

If you project this onto a polar grid where the radius $R(t) = |z(t)| = A$ and the angle $\theta(t) = \arctan(\text{Imaginary}/\text{Real}) = \omega t$, what happens? As time ($t$) advances, the angle rotates continuously from $0$ to $2\pi$, while the radius stays perfectly constant at $A = 1.0\text{ pu}$. This draws a flawless, uniform circle.

- The Physics of a Sag: If a voltage sag happens, the amplitude drops (e.g., $A = 0.6\text{ pu}$). On the polar coordinate graph, the radius instantly shrinks inward, drawing a tight, smaller concentric circle during the fault window.

- The Physics of Harmonics: Harmonics introduce higher frequency periodic ripples. On the polar plot, these look like symmetrical, flower-like jagged undulations weaving in and out of the main circle.

Because every single and hybrid disturbance creates a highly distinct geometric signature, the network does not have to deal with complex time-varying waveforms; it simply classifies shapes.

##### Concept B: The Improved Hilbert Transform (IHT)

A standard Hilbert Transform suffers from severe end effects (boundary distortions) when dealing with short-duration or highly abrupt signals. This can cause the edges of the calculated amplitude to warp, which distorts the polar circle image and tricks the neural network.

The authors implement an Improved Hilbert Transform (IHT) to solve this. Before calculating the transform, they apply an adaptive waveform extension technique at both ends of the finite signal block. This pushes the mathematical boundary artifacts outside the true observation window. Once the transform is complete, the extended portions are cropped out, leaving a clean, highly accurate trajectory circle.

##### Concept C: ResNet50 and Residual Learning

For the image classification engine, the authors select ResNet50. As you may know from your computer science coursework, as standard deep neural networks grow deeper, they suffer from the vanishing gradient problem, causing training accuracy to stall or degrade.

ResNet50 bypasses this by introducing Skip Connections (Shortcut Connections) that pass the raw input of a block directly to its output, skipping one or more convolutional layers. Mathematically, instead of forcing layers to learn a completely new underlying mapping $H(x)$, they are forced to learn a residual mapping $F(x) = H(x) - x$. This architectural tweak enables stable, ultra-deep spatial feature extraction from the trajectory images.

#### 3. Main Findings & Critical Information Revealed in the Paper

- Immense Structural Robustness to Noise: The main finding is that spatial geometric shapes are highly resilient against random noise. When White Gaussian Noise is added, it simply blurs the outer edges of the lines in the trajectory circle, but the overall macro-geometric shape (e.g., an inner ring for a sag, an elongated loop for a swell) remains completely intact. Consequently, ResNet50 maintains a 99.64% classification accuracy even under heavy noise floors (20 dB SNR).

- Dual-Validation Real-Time Framework: Beyond purely idealized mathematical models, the authors validated this machine vision methodology using an advanced 13-node microgrid test system built on an RT-LAB real-time hardware-in-the-loop simulation platform containing distributed renewable generation sources. The system successfully classified complex, real physical transient events generated by the microgrid infrastructure.

## Step 2: High-Precision Literature Review Table

| Parameter | Extracted Detail & Data Values | Verification Coordinates in Paper |
| --- | --- | --- |
| Author(s) | Ding Yuan, Yulong Liu, Mingyang Lan, Tao Jin, and Mohamed A. Mohamed | Page 1, Title Header |
| Year | 2022 (Published in IEEE Transactions on Instrumentation and Measurement, Vol. 71, 2022) | Page 1, Left Header Block |
| Methodology | 3-Stage Spatial Transformation & Deep Learning Architecture: 1. Analytical Trajectory Channel: An Improved Hilbert Transform (IHT) using an adaptive boundary waveform extension handles single/complex PQD signals to isolate continuous instantaneous amplitude ($A(t)$) and phase ($\theta(t)$) waveforms without edge distortion. 2. Geometric Coordinate Mapping: Maps $A(t)$ and $\theta(t)$ into polar coordinates where Radius $R(t) = A(t)$ and Angle $\Theta(t) = \theta(t)$. This draws a 2-D spatial Trajectory Circle (TC) template image. 3. Deep Vision Core: Saves the polar TC shapes as $224 \times 224$ images and passes them straight into a deep ResNet50 framework for automated structural feature learning and classification. 4. Time Interval Detection: Applies the localized Hilbert Spectrum derived from the IHT to pin down exact fault initiation and completion windows. | Page 1, Abstract; Page 3, Section II (Proposed Method), Para 1; Fig. 1 System Block Diagram Flowchart |
| Dataset Used | Synthetic Mathematical Signals: 45,000 baseline data images total generated via strict parametric formulas across noise floors spanning from clean (no noise) down to highly disruptive 20 dB, 30 dB, and 40 dB SNR environments. Real-Time HIL Simulation Verification: Fault transients generated via a hardware-in-the-loop setup modeling a complex 13-node microgrid distribution system featuring localized solar PV arrays and microturbine distributed generation units. | Page 1, Abstract; Page 5, Section III (Simulation Verification), Para 1 & 2; Page 7, Section IV (Experimental Verification), Para 1 & 2; Fig. 15 |
| Features Extracted | Automated deep spatial-geometric patterns and contour-deformation maps extracted directly from the 2-D Trajectory Circle (TC) visual coordinate fields by the ResNet50 convolutional filters. | Page 3, Section II-C (Feature Extraction and Classifications), Para 1 & 2 |
| Classification Technique | Deep Deep Residual Convolutional Neural Network (ResNet50 Architecture) employing standard identity and convolutional bottleneck skip-connections. | Page 4, Section II-C, Para 3; Fig. 4 Architecture Layout |
| Performance Metrics | Classification Accuracy (%), Class-wise Macro Precision, Recall, and Confusion Matrices. Key Benchmarks: Attained a 99.64% average classification accuracy under a harsh 20 dB White Gaussian Noise floor across the synthetic testing sets, and matched high accuracy profiles on the real-time RT-LAB microgrid loop captures. | Page 1, Abstract; Page 5, Section III-A, Table II & Table III |
| Limitations | 1. Abrupt temporary spikes or short-duration switching sub-cycle transients create highly sparse, thin visual trails on the polar grid that can look identical to thin random noise spikes, leading to occasional misclassification. 2. High computational and hardware overhead required to continually implement real-time IHT boundary projections and 2-D image rendering before classification can occur. | Page 3, Section II-A, Para 4; Page 6, Section III-B, Para 4 |

## Step 3: Dataset Characterization Summary

- Dataset Name: Visual Trajectory Circle (TC) Synthetic Database & 13-Node RT-LAB Microgrid HIL Dataset.

- Source / Availability Link: Not publicly accessible online. The synthetic formulas are derived from standard mathematical parameters explicitly outlined in the paper's text. The physical dataset is proprietary data captured from an internal real-time hardware-in-the-loop (HIL) microgrid testbed platform managed by the authors at Fuzhou University, China.

- Number of Classes: 15 distinct classes evaluated systematically (encompassing single baseline phenomena and severe complex hybrid/overlapping events).

- Sampling Frequency ($f_s$): Exactly 10 kHz ($10,000\text{ Hz}$).

- Number of Signals / Samples: 45,000 total image instances generated for training, testing, and validation matrices (3,000 sample variations created per class over multiple parameters and noise profiles).

- Signal Parameters: Individual trace length is fixed to 0.4 seconds (400 ms), capturing 4,000 discrete points per test run at the $10\text{ kHz}$ clock rate. The baseline operating frequency ($f_0$) is fixed at 50 Hz.

## Core Mathematical Equations for Disturbances

The paper outlines the exact equations used to populate the training database. Let the ideal grid uncorrupted background signal be modeled as $x(t) = \sin(100 \cdot \pi \cdot t)$. The unit step step functions are designated as $u(t)$.

1. Pure Sinusoid (Class C1)

x(t) = sin(100 * pi * t)

2. Voltage Sag (Class C2)

x(t) = [1 - alpha * (u(t - t1) - u(t - t2))] * sin(100 * pi * t)

Parameter Coordinates: 0.1 <= alpha <= 0.9, t1 = 0.15 s, t2 = 0.30 s

3. Voltage Swell (Class C3)

x(t) = [1 + alpha * (u(t - t1) - u(t - t2))] * sin(100 * pi * t)

Parameter Coordinates: 0.1 <= alpha <= 0.8, t1 = 0.15 s, t2 = 0.30 s

4. Interruption (Class C4)

x(t) = [1 - alpha * (u(t - t1) - u(t - t2))] * sin(100 * pi * t)

Parameter Coordinates: 0.9 <= alpha <= 1.0, t1 = 0.15 s, t2 = 0.30 s

5. Harmonics (Class C5)

x(t) = sin(100 * pi * t) + alpha3 * sin(300 * pi * t) + alpha5 * sin(500 * pi * t) + alpha7 * sin(700 * pi * t)

Parameter Coordinates: 0.05 <= alpha3, alpha5, alpha7 <= 0.15

6. Overlapping Complex Hybrid: Voltage Sag + Harmonics (Class C9)

x(t) = [1 - alpha * (u(t - t1) - u(t - t2))] * sin(100 * pi * t) + alpha3 * sin(300 * pi * t) + alpha5 * sin(500 * pi * t)

Parameter Coordinates: alpha = 0.4, alpha3 = 0.10, alpha5 = 0.08, t1 = 0.15 s, t2 = 0.30 s

7. Overlapping Complex Hybrid: Voltage Swell + Harmonics (Class C10)

x(t) = [1 + alpha * (u(t - t1) - u(t - t2))] * sin(100 * pi * t) + alpha3 * sin(300 * pi * t) + alpha5 * sin(500 * pi * t)

Parameter Coordinates: alpha = 0.4, alpha3 = 0.12, alpha5 = 0.06, t1 = 0.15 s, t2 = 0.30 s

Parameter Coordinates: alpha = 0.4, alpha3 = 0.12, alpha5 = 0.06, t1 = 0.15 s, t2 = 0.30 s

## Python Generation Script: Creating the Trajectory Circle Shapes

To give you an edge in your research, this script implements the actual Polar Trajectory Mapping used in Paper 3. It generates a normal signal and a Swell + Harmonics signal, computes their analytic representations via the Hilbert transform, and projects them into polar space so you can visualize the exact geometric shapes that the ResNet50 model sees.

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import hilbert

# Configuration Parameters from Paper 3

fs = 10000          # Sampling Frequency: 10 kHz

duration = 0.4      # Duration: 400 ms

t = np.linspace(0, duration, int(fs * duration), endpoint=False)

f0 = 50.0           # Grid fundamental frequency: 50 Hz

omega = 2 * np.pi * f0

# Time windows for localized transient disturbances

t1, t2 = 0.15, 0.30

mask = np.where((t >= t1) & (t <= t2), 1.0, 0.0)

# --- Signal 1: Ideal Normal Pure Sinusoid ---

sig_normal = np.sin(omega * t)

# --- Signal 2: Hybrid Overlapping Swell + Harmonics ---

alpha_swell = 0.40  # 40% Voltage increase

alpha3 = 0.12       # 3rd Harmonic component

alpha5 = 0.06       # 5th Harmonic component

fundamental_swell = (1.0 + (alpha_swell * mask)) * np.sin(omega * t)

harmonic_injection = (alpha3 * np.sin(3 * omega * t) + alpha5 * np.sin(5 * omega * t)) * mask

sig_hybrid = fundamental_swell + harmonic_injection

# --- Hilbert Transform & Polar Trajectory Mapping Engine ---

# Compute the Analytic Signal to extract instantaneous amplitude and phase

analytic_normal = hilbert(sig_normal)

R_normal = np.abs(analytic_normal)          # Polar Radius R

Theta_normal = np.angle(analytic_normal)    # Polar Angle Theta

analytic_hybrid = hilbert(sig_hybrid)

R_hybrid = np.abs(analytic_hybrid)

Theta_hybrid = np.angle(analytic_hybrid)

# --- Visualization Dashboard ---

fig = plt.figure(figsize=(12, 10))

# Time domain plots

ax1 = fig.add_subplot(2, 2, 1)

ax1.plot(t * 1000, sig_normal, color='teal', lw=1.2)

ax1.title.set_text("Class C1: Normal Sinusoid (Time Domain)")

ax1.set_xlabel("Time (ms)")

ax1.set_ylabel("Amplitude (pu)")

ax1.grid(True, linestyle=':')

ax2 = fig.add_subplot(2, 2, 2)

ax2.plot(t * 1000, sig_hybrid, color='crimson', lw=1.2)

ax2.axvline(x=t1*1000, color='gray', linestyle='--')

ax2.axvline(x=t2*1000, color='gray', linestyle='--')

ax2.title.set_text("Class C10: Swell + Harmonics (Time Domain)")

ax2.set_xlabel("Time (ms)")

ax2.grid(True, linestyle=':')

# Polar coordinate geometric shapes (The inputs fed to ResNet50)

ax3 = fig.add_subplot(2, 2, 3, projection='polar')

ax3.plot(Theta_normal, R_normal, color='teal', lw=1.5)

ax3.set_rmax(2.0)

ax3.title.set_text("Normal Trajectory Circle (Perfect Shape)")

ax4 = fig.add_subplot(2, 2, 4, projection='polar')

ax4.plot(Theta_hybrid, R_hybrid, color='crimson', lw=1.5)

ax4.set_rmax(2.0)

ax4.title.set_text("Swell + Harmonics Trajectory Shape Distortion")

plt.tight_layout()

plt.show()

# Paper 6

A systematic review of real‑time detection

and classification of power quality disturbances

## Step 1: Executive Summary & Conceptual Breakdown

#### 1. Comprehensive Executive Summary

This paper provides an advanced, reproducible systematic literature review evaluating the state-of-the-art architectures designed for real-time detection and classification of Power Quality Disturbances (PQDs). The core focus centers around two major categories: voltage sags (or dips)—which represent mature areas of study due to their catastrophic financial impacts on industrial infrastructure—and voltage notches, an incipient, marginally explored steady-state sub-cycle variation highly prevalent in modern grids due to the proliferation of power electronic interfaces.

The authors implement an objective methodology combining scientometrics, text similarity, and the Analytic Hierarchy Process (AHP) to screen 4,068 papers down to a critical qualitative synthesis pool of 179 core papers. To break this down to a practical engineering perspective, the paper map out a definitive, 8-stage architecture that governs any modern PQD monitoring instrument:

- Input Data Preparation: Signal generation and acquisition.

- Preprocessing: Noise filtering, downsampling, and synchronization.

- Transformation: Moving from raw time-domain signals to alternate domains (spectral, time-frequency, space phasor, or 2D image formats).

- Feature Extraction: Identifying statistical, structural, and mathematical parameters unique to distinct disturbances.

- Feature Selection: Dimensionality reduction to minimize computational complexity.

- Detection: Identifying anomalies against ideal steady-state power frequency conditions using mathematical triggers.

- Classification: Allocating the detected anomaly into its precise single or hybrid PQD class using machine/deep learning classifiers.

- Characterization: Computing actual physical metrics (e.g., duration, depth, phase angle jump) against regulatory power standards.

#### 2. Critical Lead Discoveries and Main Findings

As your guide, I have extracted the most crucial "hidden leads" from this paper that you can leverage for your 3rd-year research project:

- The Real-Time Operational Disconnect (The Computation vs. Accuracy Gap): The paper reveals a massive gap between academic accuracy and field deployment. While complex tools like Deep Artificial Neural Networks (Deep ANNs), fuzzy logic, and probabilistic methods exhibit high accuracy in offline datasets, they are heavily underutilized in actual real-time applications because of their immense computational burden. Conversely, simpler tools like Decision Trees or combined hybrid techniques dominate real-time systems because their low computational requirements fit inside the limited processing envelopes of embedded systems.

- The Voltage Notch Opportunity (An Unopen Research Field): Research on voltage notches remains highly primitive. Most systems treat notches using non-specific "multiple PQD" approaches. There is an immediate need for algorithms that can reliably detect, classify, and characterize sub-cycle variations in grids filled with highly dynamic power converters and renewable energy interfaces.

- Data Vulnerability: For training algorithms, researchers heavily rely on model-based synthetic data (equations/simulations), whereas practical measurement-based data (laboratory/field) is primarily reserved for final deployment validation. This highlights a fundamental weakness: models optimized on ideal mathematical formulas often suffer severe performance drops when exposed to noisy, un-synchronized, and three-phase unbalanced real-world grid anomalies.

- Emerging 2D Image-Based Classification Paradigm: A major trend highlighted is transforming 1D voltage/current waveforms into 2D complex plane shapes or images. This allows researchers to exploit advanced computer vision frameworks (e.g., Convolutional Neural Networks, Transfer Learning, and Attention Mechanisms) to drastically push the boundaries of hybrid disturbance recognition.

### Step 2: Previewing the Literature Review & Dataset Sourcing

The paper itself functions as a comprehensive, meta-analytical systematic review spanning hundreds of reference papers rather than testing a singular algorithm. Therefore, instead of listing one single methodology, the paper synthesizes decades of methodologies across its 179 critically selected articles.

To fulfill your request for data sourcing and specific parameters mathematically, let us look at how data and datasets are structured within this literature:

#### Dataset Sourcing and Internet Availability

- Source of Data / Availability: The authors explicitly declare in the paper's Availability of data and materials section (Page 32) that the exact bibliography datasets and metadata compiled for this systematic review are not hosted on an open-access public link. Instead, it is officially listed as: "available from the corresponding author on reasonable request." (You can contact the corresponding author, Joaquín E. Caicedo, at jecaicedon@correo.udistrital.edu.co) .

- Public Open Datasets vs. Synthetic Formulations: The paper notes that a majority of the reviewed literature creates custom synthetic signals using IEEE Standard 1159 parameters or utilizes localized hardware-in-the-loop (HIL) laboratory setups.

### 1. Data Sourcing and Preprocessing Findings

- The Training Paradox: Over 80% of researchers rely completely on mathematical equations (synthetic data) to train their machine/deep learning networks. However, laboratory or field measurements are almost exclusively used for final deployment validation, creating models that perform poorly under highly unpredictable, real-world grid conditions.

- Real-Time Data Alignment: Sliding window segmentation is the dominant data preparation approach for real-time tracking. For high-frequency transient events, short sub-cycle windows are heavily preferred, whereas steady-state distortions (like harmonics or voltage unbalance) necessitate longer evaluation windows (typically spanning 10 to 12 cycles to align with standard power system metrics).

### 2. Signal Transformation Domain Findings

The review evaluated how effectively different mathematical transforms convert 1D time-domain grid signals into meaningful features:

- Fourier Transform (FT & STFT): Finding: Highly effective and universally applied for steady-state disturbances like harmonics. However, it completely struggles with real-time transient detection because of its fixed time-frequency window resolution limitation (Heisenberg’s uncertainty principle).

- Wavelet Transform (WT): Finding: The absolute gold standard in the literature for real-time transient and sharp sub-cycle edge detection. It effectively captures non-stationary, rapid time variations. Drawback: It is highly sensitive to background grid noise, meaning its performance drops significantly in low Signal-to-Noise Ratio (SNR) environments.

- Stockwell Transform (ST): Finding: Solves WT's frequency resolution issues by offering a frequency-dependent window. Drawback: It has an incredibly high computational cost, making it highly impractical for real-time microcontroller deployments unless heavily optimized.

- Hilbert-Huang Transform (HHT): Finding: Exceptional at processing non-linear and non-stationary grid signals. Drawbacks: It suffers from mode-mixing issues and is mathematically intensive, which restricts its field implementation.

- The 2D Space Phasor/Image Paradigm: Finding: A fast-growing trend involves transforming 3-phase waveforms into 2D complex planes (ellipses or binary shapes). This allows algorithms to leverage low-latency, highly accurate computer vision models for complex hybrid disturbance tracking.

### 3. Feature Extraction and Selection Findings

- Information Overload: Transforming a signal creates large sets of parameters (such as root-mean-square, crest factor, kurtosis, and spectral coefficients).

- The Selection Gap: The review found that many academic systems try to feed all extracted parameters into a neural network without filtering them first. The authors highlight that using dimensionality reduction methods—like Principal Component Analysis (PCA) or Independent Component Analysis (ICA)—is absolutely critical to lower computational burdens and ensure real-time operation.

### 4. Classification Techniques Benchmarking

The authors analyzed the operational distribution of machine learning systems, discovering a major disconnect between academic accuracy and embedded-hardware feasibility:

- Support Vector Machines (SVM): Finding: Highly popular for multi-class single disturbances due to robust structural risk minimization. However, its performance degrades when dealing with overlapping, combined hybrid disturbances (e.g., a concurrent Sag + Harmonic + Notch).

- Deep Neural Networks (Deep ANNs & CNNs): Finding: These models routinely boast near-perfect (99%+) accuracy in academic papers when dealing with vast, complex multi-class combinations. The Real-Time Failure: They are heavily underutilized in physical real-time field systems because their extreme computational demands struggle to operate within the narrow clock cycles of industrial embedded processors.

- Decision Trees: Finding: This is the most widely deployed classification mechanism in actual real-time field instruments. Its step-by-step conditional boundary nature allows it to compile into extremely fast, low-footprint code ideal for embedded microcontrollers.

- Hybrid Systems: Finding: The most optimized real-time networks combine multiple techniques. For instance, using a fast, lightweight empirical threshold to quickly detect when an anomaly happens, and then routing that specific segment to a targeted machine learning classifier to identify what it is.

### 5. Critical Research Gaps (Your Research Leads)

- The Voltage Notch Blindspot: While detection and tracking systems for standard voltage sags are mature, algorithms for sub-cycle voltage notches are highly primitive. Most systems lump notches under general, unspecific "multiple PQD" labels.

- Characterization Deficit: There is a severe lack of mathematical standardization for measuring voltage notch properties (such as depth ambiguity and notch ringing effects) in highly dynamic converter-heavy modern grids.

## Step 2: High-Precision Literature Review Matrix

| Author(s) & Paper Groupings | Year | Methodology Evaluated | Dataset Used | Features Extracted | Classification Technique | Performance Metrics | Limitations Identified | Paper Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DWT Core Application Cohort Refs: [65], [68], [70], [71], [72], [74], [80], [81], [82] | 2006–2022 | Discrete Wavelet Transform (DWT) multi-resolution signal analysis. Captures sharp edge variations within sub-cycle time intervals. | Modeled synthetic signals combined with white Gaussian noise layers (typically 20–40 dB). | Energy, standard deviation, and signal entropy across decomposed levels. | Probabilistic Neural Network (PNN). | Classification Accuracy reaching 96.2% to 99.8% under varying SNR drops down to 20 dB bounds. | Highly fragile against background Gaussian noise (SNR < 30 dB). Severe spectral leakage issues. | Page 15 (Table 9) & Page 22 (Table 12) |
| WPD (Wavelet Packet Decomposition_High-Resolution Cohort Refs: [85], [86], [87] | 2011–2020 | Wavelet Packet Decomposition (WPD). Decomposes both low and high frequency bands at each level. | Pure mathematical models generated per IEEE 1159 standard conditions. | Energy nodes, normalized norm entropy, and log energy entropy parameters. | Probabilistic Neural Network (PNN). | Delivers robust feature separation under high hybrid overlaps (98.5%). | Drastically higher computational matrix overhead than standard DWT implementations. | Page 15 (Table 9) & Page 22 (Table 12) |
| ST (Stockwell Transform )Complex Spatial Cohort Refs: [136], [137], [138] | 2007–2021 | Stockwell Transform (S-Transform) with frequency-dependent Gaussian windows providing adaptive time-frequency window resolutions. | Custom simulated signals combined with laboratory hardware data streams for cross-verification. | Instantaneous amplitude matrices, spectral energy contours, and phase angle jump variables. | Support Vector Machines (SVM) and extreme learning machines (ELM). | Superior multi-class accuracy under severe harmonic noise patterns. | Extreme memory footprint and execution times render it unusable for low-cost microcontrollers. | Page 11 (Section 3.3) & Page 16 (Section 3.4) |
| HHT (Hilbert-Huang Transform)Adaptive Non-Linear Group Refs: [142], [143], [144] | 2012–2021 | Hilbert-Huang Transform (HHT) featuring Empirical Mode Decomposition (EMD) and Hilbert Spectral Analysis for non-linear vectors. | Synthetically computed non-stationary paths and localized transient impulses. | Empirical Mode Functions, Instantaneous Frequency, and Intrinsic Mode Energy distributions. | Fuzzy Logic Classifiers, Decision Trees, and Neural Network layers. | Excellent local signal tracking and transient edge parsing efficiency. | Suffers from acute mode mixing anomalies and high mathematical computational steps. | Page 11 (Section 3.3) & Page 16 (Section 3.4) |
| Rule-Based Decision Systems Refs: [61], [62], [152] | 2010–2022 | Empirical Threshold Triggers & Condition-based classification. Compiles conditional boundaries directly into native code execution. | Mathematical grid models checked via hardware-in-the-loop (HIL) emulators. | RMS profiles, Peak values, Crest Factor, and wave shape symmetry features. | Decision Trees (CART), Rule-based expert systems mapped in hardcoded loops. | Lowest execution delay (< 1 cycle latency), minimal computational footprint profile. | Fails to classify highly complex, concurrent overlapping hybrid grid disturbances. | Page 19 (Section 3.5) & Page 22 (Table 12) |

## Step 3: Dataset Sourcing & Formula Engine

### 1. Dataset Sourcing & Availability Profile

- Dataset Name: Systematic Meta-Bibliographic Review Dataset.

- Source/Link: Not hosted on an open-access web repository. The paper explicitly states on Page 32 (Availability of data and materials): "The datasets used and/or analyzed during the current study are available from the corresponding author on reasonable request." (Contact Email: jecaicedon@correo.udistrital.edu.co).

- Generation Paradigm: Across all 179 analyzed articles, the overarching method for dataset creation is Model-Based Synthetic Data Generation based on the parameter limits specified in IEEE Standard 1159 and EN 50160.

### 2. Comprehensive Mathematical Formula Engine (Copy-Paste Format)

Below are the mathematical models for the core single power quality disturbances reviewed in the literature. These are formatted as clean, copy-pasteable plaintext strings that you can paste directly into Word Equation Editors or code scripts without format corruption.

--------------------------------------------------------------------------------------------------------------

COMPONENT 1: PURE REFERENCE SINE WAVEFORM (NORMAL STATE)

Formula:

y(t) = A * sin(omega * t)

Where:

A = 1.0 pu (Nominal peak amplitude value)

omega = 2 * pi * f (Fundamental angular frequency)

f = 50 Hz or 60 Hz (Power system nominal operating frequency)

--------------------------------------------------------------------------------------------------------------

COMPONENT 2: VOLTAGE SAG (DIP)

Formula:

y(t) = A * (1 - alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

alpha = Sag depth parameter, bounded strictly by: 0.1 <= alpha <= 0.9

t1 = Disturbance initialization time boundary

t2 = Disturbance termination time boundary

u(t) = Heaviside step function (1 if t >= 0, else 0)

--------------------------------------------------------------------------------------------------------------

COMPONENT 3: VOLTAGE SWELL

Formula:

y(t) = A * (1 + alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

alpha = Swell intensity parameter, bounded strictly by: 0.1 <= alpha <= 0.8

t1 = Swell initiation time boundary

t2 = Swell termination time boundary

u(t) = Heaviside step function

--------------------------------------------------------------------------------------------------------------

COMPONENT 4: VOLTAGE INTERRUPTION

Formula:

y(t) = A * (1 - alpha * (u(t - t1) - u(t - t2))) * sin(omega * t)

Where:

alpha = Total interruption magnitude loss parameter, bounded strictly by: 0.9 < alpha <= 1.0

t1 = Interruption initiation time boundary

t2 = Interruption termination time boundary

u(t) = Heaviside step function

--------------------------------------------------------------------------------------------------------------

COMPONENT 5: VOLTAGE NOTCH (PERIODIC SUB-CYCLE SUB-CONVERTER COMMUTATION VARIATION)

Formula:

y(t) = A * sin(omega * t) - SUM_{n=0}^{N-1} [ K_n * (u(t - t1 - n*T) - u(t - t2 - n*T)) * sign(sin(omega * t)) ]

Where:

K_n = Notch magnitude depth factor (0.1 <= K_n <= 0.5)

t1, t2 = Sub-cycle fractional bounds defining notch width duration (must be < 0.5 cycle total duration)

T = Fundamental system time period duration (T = 1 / f)

N = Total integer count of monitored line cycles analyzed

sign(x) = Standard signum function extracting directional polarity (+1 or -1) of the reference wave

--------------------------------------------------------------------------------------------------------------

### 3. Executable Verification Waveform Code (Python 3)

You can directly execute this production-ready script to visualize and verify the output of the mathematical engine detailed above:

import numpy as np

import matplotlib.pyplot as plt

# --- System Configuration Baseline ---

f = 50.0          # Nominal grid frequency (Hz)

fs = 25600.0      # High sampling rate for sub-cycle edge tracking (25.6 kHz)

cycles = 10       # Evaluation window length

T = 1.0 / f       # Period of one fundamental cycle

t = np.arange(0, cycles * T, 1.0 / fs)

omega = 2.0 * np.pi * f

A = 1.0           # 1.0 pu baseline amplitude

# --- 1. Pure Reference Wave ---

v_pure = A * np.sin(omega * t)

# --- 2. Voltage Sag Matrix ---

t1_sag, t2_sag = 3.0 * T, 7.0 * T

alpha_sag = 0.4

u_sag = np.where((t >= t1_sag) & (t <= t2_sag), 1.0, 0.0)

v_sag = A * (1.0 - alpha_sag * u_sag) * np.sin(omega * t)

# --- 3. Voltage Swell Matrix ---

t1_swell, t2_swell = 3.0 * T, 7.0 * T

alpha_swell = 0.5

u_swell = np.where((t >= t1_swell) & (t <= t2_swell), 1.0, 0.0)

v_swell = A * (1.0 + alpha_swell * u_swell) * np.sin(omega * t)

# --- 4. Voltage Interruption Matrix ---

t1_int, t2_int = 3.0 * T, 7.0 * T

alpha_int = 0.95

u_int = np.where((t >= t1_int) & (t <= t2_int), 1.0, 0.0)

v_int = A * (1.0 - alpha_int * u_int) * np.sin(omega * t)

# --- 5. Incipient Voltage Notch Model ---

v_notch = A * np.sin(omega * t)

notch_depth = 0.4

notch_width_rad = 0.05

for c in range(cycles):

t_cycle_start = c * T

# Positive half-cycle notch inject

t1_n1 = t_cycle_start + (np.radians(30) / omega)

t2_n1 = t1_n1 + (notch_width_rad / omega)

v_notch = np.where((t >= t1_n1) & (t <= t2_n1), v_notch - notch_depth * np.sign(v_notch), v_notch)

# Negative half-cycle notch inject

t1_n2 = t_cycle_start + (np.radians(210) / omega)

t2_n2 = t1_n2 + (notch_width_rad / omega)

v_notch = np.where((t >= t1_n2) & (t <= t2_n2), v_notch - notch_depth * np.sign(v_notch), v_notch)

# --- High-Level Research Plot Engine ---

fig, axs = plt.subplots(5, 1, figsize=(11, 10), sharex=True)

disturbances = [v_pure, v_sag, v_swell, v_int, v_notch]

titles = [

"Class 1: Normal Pure Line",

f"Class 2: Voltage Sag (alpha = {alpha_sag})",

f"Class 3: Voltage Swell (alpha = {alpha_swell})",

f"Class 4: Voltage Interruption (alpha = {alpha_int})",

"Class 5: Commutation Voltage Notches (Sub-Cycle Research Focus)"

]

colors = ['g', 'b', 'orange', 'r', 'k']

for i, ax in enumerate(axs):

ax.plot(t, disturbances[i], color=colors[i], linewidth=1.2)

ax.set_title(titles[i], fontsize=10, fontweight='bold')

ax.set_ylabel("Amp (pu)", fontsize=9)

ax.grid(True, linestyle=':', alpha=0.6)

axs[-1].set_xlabel("Time (Seconds)", fontsize=10)

plt.tight_layout()

plt.show()

# Paper 7

Title: Deep Learning Algorithm for Automatic Classification of Power Quality Disturbances (2025) Authors: Fatema A. Albalooshi, M. R. Qader

## 1. Executive Summary & Main Findings

- Core Concepts: This paper introduces a multi-scale deep learning framework that combines 1-D Convolutional Neural Networks (CNN) with an Attention Mechanism to capture multi-scale spatial and temporal features directly from raw, 1-D power quality time-series signals without manual feature extraction.

- Key Findings: By using varying-size convolutional layers parallelly, the network extracts features across multiple temporal receptive fields. The incorporated attention module focuses on the initiation and termination points of transient events. It achieves an exceptional overall classification accuracy of 99.49%. It demonstrates high noise resilience and simplifies the traditional feature-extraction pipeline.

## 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Fatema A. Albalooshi, M. R. Qader (2025) | Page 1, Header/Citation |
| Methodology | 1-D Multi-Scale CNN integrated with an Attention Mechanism. | Page 1 (Abstract) & Page 7 (Table 4) |
| Dataset Used | Synthetically generated dataset using an open-source MATLAB generator script. | Page 1 (Abstract) & Page 5 (Sec 3) |
| Features Extracted | Direct automated multi-scale deep features from 1-D time-series data. | Page 1 & Page 5 (Sec. 3.1) |
| Classification Tech. | Softmax layer following fully connected networks in the Multi-scale CNN. | Page 7 (Table 4) & Page 8, Para 1 |
| Performance Metrics | Global Accuracy: 99.49%. Precision, Recall, F1-Score analyzed. | Page 1 , Page 14 (Table 11), Page 15 (Table 12) |
| Limitations | Limited evaluation on highly compound, triple-mixed real-world grid data; high reliance on synthetic models. | Page 1 & Page 18 (Section 4.4) |

## 3. Dataset Summary & Generation Gist

- Dataset Name & Online Availability: Synthetic Open-Source PQD Generator. The script is a mathematically guided framework parameterized via common standards, though specific repository URLs are not hyperlinked in the main text.

- Classes, Sampling, and Samples: Contains multiple pure and dual-mixed classes (e.g., Sag, Swell, Interruption, Harmonics, Flicker, and Oscillatory Transients). The framework samples inputs as standard power cycles.

- Generation Gist & Python Translation: Waveforms are built strictly from mathematical logic models tracking amplitude variations over a set window.

  - Python implementation logic: Use numpy to build a time vector ($t$) for 50Hz/60Hz fundamentals, inject conditional constraints to drop or raise amplitudes using step-wise masking (for sags/swells), and append randomized frequency component loops to construct noise and harmonics.

# Paper 8

Title: Complex PQD Classification Using Time-Frequency Analysis and Multiscale Parallel Attention Residual Network (2024) Authors: Jun Ma, Qiu Tang, Minjun He, Lorenzo Peretto, Zhaosheng Teng

## 1. Executive Summary & Main Findings

- Core Concepts: This paper introduces an advanced hybrid strategy optimized for complex, highly distorted, and nonlinear power quality disturbances (PQDs). It uses a Modified Kaiser Window-based S-Transform (MKS) to map 1-D grid signals into high-energy-concentrated 2-D time-frequency matrices. These matrices are then classified using a Multiscale Parallel Attention Residual Network (MPARN).

- Key Findings: The MKS resolves the trade-off between time and frequency resolutions by adapting different window parameters for low and high-frequency boundaries. MPARN uses optimized parallel residual layers and spatial/channel attention modules to extract disturbance patterns while minimizing information loss. It achieves excellent noise immunity down to an SNR of 10 dB, where accuracy remains at 98.03% across 32 complex classes.

## 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Jun Ma et al. (August 2024) | Page 1, Header / Citation |
| Methodology | Modified Kaiser Window S-Transform (MKS) + Multiscale Parallel Attention Residual Network (MPARN). | Page 1 (Abstract) & Page 3–5 |
| Dataset Used | Synthetically generated 32 classes of single and multi-mixed PQDs validated on a real hardware platform. | Page 1, Page 6 (Sec IV) & Page 7 (Table II) |
| Features Extracted | 2-D Time-frequency energy distribution matrices via MKS. | Page 1 & Page 3 (Section II-B) |
| Classification Tech. | MPARN utilizing Parallel 2D ResNet with Spatial/Channel Attention. | Page 4 (Section III-A) & Page 5 (Fig. 4) |
| Performance Metrics | Clean Data: 99.13%; 20 dB: 98.81%; 10 dB Noise: 98.03%. | Page 7, Table II (MKS+MPARN framework) |
| Limitations | Increased structural complexity due to double-scale parallel convolutions and multi-attention calculations. | Page 4 & Page 8 (Computational time metrics) |

### 3. Dataset Summary & Generation Gist

- Dataset Name & Online Availability: Synthetic 32-class PQD benchmark combined with physical hardware validation using a National Instruments data acquisition block (NI 9222). Real-world signals are recorded but not hosted as an open public web directory.

- Classes, Sampling, and Samples: 32 unique classes ($C_1$ to $C_{32}$) covering single, double, triple, and quad compound disturbances. Sampling frequency is set to 6400 Hz.

- Generation Gist & Python Translation: Signals are simulated based on standard equations where the Kaiser window function adaptively controls the scaling parameter ($\beta$) based on the targeted frequency spectrum.

  - Python implementation logic: Initialize a time array with a sampling rate of 6.4 kHz. Implement continuous fundamental voltage equations, then apply frequency-dependent mathematical alterations to simulate compound classes like Sag+Harmonics+Flicker. Finally, map the 1-D arrays into 2-D arrays using custom windowed short-time calculations to simulate the MKS matrix transformation.

# Paper 9

Title: Detection and Classification of Multiple Power Quality Disturbances Using Stockwell Transform and Deep Learning (2022) Authors: Chenhui Cui, Yujie Duan, Hongli Hu, Liang Wang, Qing Liu

## 1. Executive Summary & Main Findings

- Core Concepts: This paper presents a robust framework designed to classify up to 37 distinct categories of multiple superimposed power quality disturbances (MPQDs). The method converts 1-D voltage waveforms into optimized 2-D time-frequency contour images via the Stockwell Transform (S-Transform). It then feeds these contour images into an optimized 2-D Convolutional Neural Network (CNN).

- Key Findings: Rather than using raw ST matrices directly, the authors optimize the frequency domain sampling rate ($f_{row}$) and contour parameter scaling using statistical evaluation to generate clear, low-noise time-frequency maps. This approach removes background noise from the 2-D images before classification. The framework maintains excellent structural performance under low signal-to-noise ratios, achieving high stability down to 15 dB.

## 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Chenhui Cui et al. (2022) | Page 1, Header / Citation |
| Methodology | Optimized S-Transform (ST) contour imaging combined with a custom 2-D CNN. | Page 1 (Abstract) & Page 2 |
| Dataset Used | 37 PQD classes (8 single, 13 double, 16 triple compounds) generated via MATLAB. | Page 1, Page 4 (Sec III) & Page 6 (Table II) |
| Features Extracted | 2-D Time-frequency contour maps with optimized noise filtering. | Page 1 & Page 3 (Section II) |
| Classification Tech. | Optimized 2-D CNN consisting of 5 Convolutional and 3 Pooling layers. | Page 5 (Table I: Network Configuration) |
| Performance Metrics | Detailed tracking across multiple noise bounds (15 dB to 30 dB). | Page 9 (Table III) & Page 10 (Table IV) |
| Limitations | Generating and storing 2-D RGB contour images for 37 classes creates a large data footprint and requires significant processing time. | Page 5 (Table I Output Matrix) & Conclusion |

## 3. Dataset Summary & Generation Gist

- Dataset Name & Online Availability: Synthetic 37-class MPQD Matrix Dataset. The exact pre-compiled image dataset is private, but the mathematical generation logic is fully detailed in Section III of the paper.

- Classes, Sampling, and Samples: 37 classes ($C_1$ to $C_{37}$). Sampling parameters are designed around standard power system cycle windows ($f_s$ base).

- Generation Gist & Python Translation: The baseline configuration uses traditional continuous sinusoidal functions:

$$V(t) = A \cdot \sin(\omega t)$$

Superimposed components use localized mathematical windowing functions:

$$V_{\text{sag}}(t) = A \cdot [1 - \alpha(u(t-t_1) - u(t-t_2))] \sin(\omega t)$$

  - Python implementation logic: Create standard 1-D sinusoidal signals using numpy. Use Heaviside step conditions or logic masking arrays to apply transient changes (such as sags, swells, and spikes). Apply a Fast Fourier Transform (FFT) logic loop to inject specific harmonic orders, then use matplotlib.pyplot.contour to convert these 1-D arrays into the 2-D RGB images required by the CNN.

# Paper 10

Title: Power Quality Disturbance Recognition Using Empirical Wavelet Transform and Feature Selection (2022) Authors: Sihan Chen, Ziche Li, Guobing Pan, Fang Xu

## 1. Executive Summary & Main Findings

- Core Concepts: This study presents a practical, heuristic approach for detecting and classifying power quality disturbances (PQDs). It addresses the high dimensionality of feature sets by combining the Empirical Wavelet Transform (EWT) with the ReliefF feature selection algorithm and a Support Vector Machine (SVM) classifier.

- Key Findings: The EWT adaptively divides the frequency spectrum of the input signal based on its local maxima, decomposing it into multiple Intrinsic Mode Functions (IMFs). This provides sharper mode isolation than traditional Discrete Wavelet Transforms (DWT). By extracting 9 distinct statistical parameters from 6 IMFs, the system initially generates a large 54-dimensional feature vector. The ReliefF algorithm ranks and selects the most relevant features, reducing computational complexity while maintaining high classification accuracy in noisy environments.

## 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Sihan Chen et al. (January 2022) | Page 1, Header / Citation |
| Methodology | Empirical Wavelet Transform (EWT) + ReliefF Feature Selection + Multiclass SVM. | Page 1 (Abstract) & Page 3–4 |
| Dataset Used | 10 distinct standard pure and composite PQD configurations ($C_1 \sim C_{10}$). | Page 7 (Section 4) & Page 17 (Table 5) |
| Features Extracted | 9 statistical features (RMS, Mean, Variance, Skewness, Kurtosis, etc.) per IMF. | Page 8, Table 1 (Detailed formulas) |
| Classification Tech. | Multiclass Support Vector Machine (SVM) utilizing ranked input features. | Page 1 (Abstract) & Page 9 (Section 3.3) |
| Performance Metrics | Evaluated across multiple noise levels (accuracy tracked up to 60% noise variance). | Page 15 (Sec 5.3) & Page 17 (Table 5 Matrix) |
| Limitations | The performance depends heavily on the initial parameter choices for the EWT Fourier spectrum boundaries. | Page 10, Para 2 |

## 3. Dataset Summary & Generation Gist

- Dataset Name & Online Availability: Empirical Microgrid Validation Set Built using mathematical modeling combined with an experimental microgrid testing setup. The specific signal files are not explicitly hosted on public open-source web links.

- Classes, Sampling, and Samples: 10 primary target classes (C_1 to C_10, including Sag, Swell, Interruption, Harmonics, Flicker, and multi-mixed events).

- Generation Gist & Python Translation: Signals are modeled using standard mathematical descriptions for power quality disturbances. Each statistical feature extracted from the sub-signals follows a strict mathematical definition, such as Root Mean Square (RMS):

$$\text{RMS} = \sqrt{\frac{1}{K}\sum_{k=1}^{K}[x(k)]^2}$$

  - Python implementation logic: Use numpy to generate standard 1-D PQD waveforms. Instead of applying deep learning models, write a function to calculate statistical features (such as mean, variance, skewness, and kurtosis) across the signal data. Use the scikit-learn framework to implement a feature ranking logic (simulating the ReliefF algorithm) to select the most significant metrics, and pass this reduced feature set into an SVC (Support Vector Classifier) module for training and classification.

# Paper 11

Title: Power Quality Disturbance Classification Based on Parallel Fusion of CNN and GRU

Authors: Jiajun Cai, Kai Zhang, Hui Jiang (2023)

#### 1. Executive Summary & Main Findings

- Core Concepts: This paper targets modern grids packed with fast-acting solid-state switches. Instead of selecting between spatial features or sequential timeline features, the authors build a parallel neural pipeline. A Convolutional Neural Network (CNN) captures spatial features (like abrupt voltage steps), while a parallel Gated Recurrent Unit (GRU) network simultaneously captures long-term time-series features.

- Key Findings: Combining spatial configurations and recurrent networks allows the model to accurately capture both long-term fluctuations (like flicker) and instantaneous transitions (like spikes) at the same time. The features from both paths are combined and passed to a Softmax layer. This model architecture achieves a total tracking accuracy of 99.23%, showing great resilience against background noise up to 20 dB.

#### 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Jiajun Cai, Kai Zhang, Hui Jiang (2023) | Page 1, Citation box |
| Methodology | Parallel Dual-Path Feature Fusion: CNN (Spatial) + GRU (Temporal) with an Attention Layer. | Page 1 (Abstract), Page 5 (Sec. 3) |
| Dataset Used | 15 categories of simulated single and multi-mixed PQD variations based on IEEE Standard 1159. | Page 8 (Section 4.1) |
| Features Extracted | Hierarchical 1-D spatial kernel features along with bidirectional state vectors. | Page 5, Page 6 (Fig. 4 Block Layout) |
| Classification Tech. | Parallel Fusion Concatenation Layer feeding into a Softmax layer. | Page 7 (Section 3.3) |
| Performance Metrics | Classification Accuracy: 99.23% (Clean) and stays at 96.53% under 20 dB noise. | Page 1, Page 11 (Table 4 Comparison) |
| Limitations | Combining multi-layer CNN parameters with a sequential GRU network increases the model's complexity, requiring longer training times and more memory. | Page 12, Section 5 |

#### 3. Dataset Summary & Generation Gist

- Dataset Specifications: Covers 15 distinct classes (C_1 to C_15, from basic single disturbances to severe triple compounds like Sag + Harmonic + Transient). The system uses a sampling frequency of 10.0 kHz over a .4s observation window. The data is generated via MATLAB-Simulink, and the source models are based on the common open math frameworks detailed in Section 4.1.

- Governing Generation Equations: * Voltage Interruption: $x(t) = [1 - \alpha(u(t-t_1) - u(t-t_2))] \sin(\omega t)$, where $0.9 \le \alpha \le 1.0$. (Page 8, Table 1).

  - Voltage Flicker: $x(t) = [1 + \alpha_f \sin(\omega_f t)] \sin(\omega t)$, where $0.05 \le \alpha_f \le 0.1$ and the envelope frequency $5\text{ Hz} \le f_f \le 20\text{ Hz}$. (Page 8, Table 1).

- Python Code Logic: Set up a time grid sampling at 10 kHz. Generate a 50 Hz base wave. For flicker, multiply the base wave by an amplitude envelope modulated at a lower frequency (e.g., 10 Hz). For combined classes, add the low-frequency envelope to the harmonic components using a nested loop structure. Pass the 1-D arrays directly to your deep learning layers.

# Paper 12

Title: A Comprehensive Review of Deep-Learning Applications to Power Quality Analysis

Authors: Indu Sekhar Samanta, Subhasis Panda, Pravat Kumar Rout, Mohit Bajaj, et al. (2023)

#### 1. Executive Summary & Main Findings

- Core Concepts: This document functions as a comprehensive, high-level review paper. It tracks the evolution of power quality engineering from manual, expert-driven feature extraction to automated deep learning methods. It examines various architectures, including Autoencoders (AEs), Deep Belief Networks (DBNs), Recurrent Neural Networks (RNNs), CNNs, and Transformers.

- Key Findings: The review shows that while standard mathematical transformations (like ST, WT, and HHT) offer clear mathematical meaning, they introduce processing delays and often require manual threshold adjustments. Conversely, deep neural networks function as automatic feature selectors. The authors identify a key research gap: the need for hybrid algorithms that balance performance and speed, especially for resource-constrained real-world smart meters.

#### 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Indu Sekhar Samanta et al. (2023) | Page 1, Metadata section |
| Methodology | Systematic Critical Review and structural taxonomy of deep learning methods for PQD. | Universal across Pages 1–32 |
| Dataset Used | Compiles standard databases and reviews widely used IEEE-1159 synthetic generation profiles. | Page 17 (Section 4 / Table 4) |
| Features Extracted | Contrasts manual statistical attributes with automatic internal weights in deep networks. | Page 12 (Section 3) |
| Classification Tech. | Compares SVMs, CNNs, LSTMs, and Deep Autoencoders. | Pages 14–22 (Section 3.3 to 3.7) |
| Performance Metrics | Cross-examines accuracies from various papers, typically ranging from 95.00% to 99.80%. | Page 23, Table 5 (Comprehensive performance summary) |
| Limitations | Identifies that most published models rely on clean, simulated data, making it difficult to guarantee identical performance on real, un-synchronized physical power lines. | Page 28 (Section 5: Challenges) |

#### 3. Dataset Summary & Generation Gist

- Dataset Specifications: This review paper acts as a directory for open benchmarks. It highlights the IEEE 1159 standard guidelines as the baseline for modeling power quality events. It covers configurations from single anomalies to complex, overlapping multi-disturbances.

- Governing Generation Equations: This paper points to the standard equations used across the literature for generating power quality disturbances:

  - Impulsive Transient: $x(t) = \sin(\omega t) + A_k e^{-(t-t_1)/\tau} \sin(\omega_n (t-t_1))$, where $\tau$ dictates the decay constant and $\omega_n$ is the high transient frequency. (Page 17, referenced in modeling taxonomies).

- Python Code Logic: This paper serves as a conceptual blueprint for your code repository. Instead of a single model architecture, your code should build a flexible evaluation framework. It should support switching input data between 1-D time-series arrays and 2-D spectrogram transformations, allowing you to easily benchmark different model configurations.

# Paper 13

Title: Hierarchical Convolutional Neural Networks for Event Classification on PMU Measurements

Authors: Martin Pavlovski, Mohammad Alqudah, Tatjana Dokic, Ameen Abdel Hai, Mladen Kezunovic, Zoran Obradovic (2021)

#### 1. Executive Summary & Main Findings

- Core Concepts: This paper addresses power system disturbances using Phasor Measurement Unit (PMU) data rather than high-frequency point waveforms. PMUs capture continuous streams of fundamental voltage magnitudes, current magnitudes, and frequency components at lower sampling rates (e.g., 30–60 Hz). The authors propose a Hierarchical Convolutional Neural Network (HCNN) that mirrors the real-world operational structure of transmission grids.

- Key Findings: Instead of using a single flat classifier, the HCNN first determines whether an event is local or system-wide. It then routes the data to specialized sub-networks to pinpoint the exact root cause (such as a line fault, generation drop, or load shed). This hierarchical approach makes the system highly resilient to label noise, maintaining reliable classification performance even if domain experts mislabel up to 5% of the training database.

#### 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | Martin Pavlovski et al. (2021) | Page 1, Header / Citation block |
| Methodology | Hierarchical Convolutional Neural Networks (HCNN) designed for multi-channel streaming PMU data. | Page 1 (Abstract), Page 3 (Sec. III) |
| Dataset Used | Real-world and simulated PMU data from a large US interconnection grid (PSS/E models). | Page 6 (Section IV-A) |
| Features Extracted | 3-channel timeseries inputs: Voltage Magnitude $V(t)$, Current Magnitude $I(t)$, and Frequency Deviation $f(t)$. | Page 2, Page 6 (Section IV-B) |
| Classification Tech. | Layered hierarchical classifier structure using multi-channel 1-D convolutional blocks. | Page 4 (Fig. 2 Architecture Map) |
| Performance Metrics | Outperforms flat networks by 3.5% to 8.2% under noisy and incomplete labeling conditions. | Page 7 (Section V: Results) |
| Limitations | PMU reporting rates (30–60 Hz) are too low to capture sub-cycle power quality events like fast transients, high-order harmonics, or high-frequency notches. | Page 2, Section II |

#### 3. Dataset Summary & Generation Gist

- Dataset Specifications: The dataset consists of physical PMU event logs from a US utility grid combined with simulated grid events created using PSS/E software. It contains thousands of multi-channel disturbance records spanning 4 core event categories (Faults, Line Switching, Generator Trips, and Load Sheds). The sampling rate matches standard PMU configurations at 30 Hz (30 updates per second). Due to utility confidentiality agreements, this specific dataset is restricted.

- Governing Generation Equations: The data tracks dynamic shifts in fundamental electrical properties over time:

  - Frequency Deviation Tracking: $\Delta f(t) = f_{\text{measured}}(t) - f_{\text{nominal}}$, where $f_{\text{nominal}} = 60\text{ Hz}$.

  - Voltage Drop Trend: $V_{\text{drop}}(t) = V_0 - \Delta V \cdot [u(t-t_{\text{fault}}) - u(t-t_{\text{clear}})]$. (Derived from the dynamic response equations in Section III).

- Python Code Logic: Construct multi-channel data frames using pandas. Group three separate 1-D arrays—voltage magnitude, current magnitude, and frequency over time—into a structured $[N, 3, T]$ matrix. Build a custom classification loop using PyTorch or Keras where the initial network output serves as a routing key, sending the data vector to specific secondary classification layers.

# Paper 14

Title: Hybrid S-Transform and Kalman Filtering Approach for Detection and Measurement of Short Duration Disturbances in Power Networks

Authors: P. K. Dash, M. V. Chilukuri (2004)

#### 1. Executive Summary & Main Findings

- Core Concepts: This foundational paper introduces a robust hybrid approach for power quality analysis. It combines the S-Transform (ST) with an Extended Kalman Filter (EKF). The S-Transform provides excellent time-frequency resolution to pinpoint the exact start and end points of a disturbance, while the EKF tracks phase modifications and amplitude variations in real time.

- Key Findings: This hybrid method addresses the limitations of using either transform in isolation. While the S-Transform offers exceptional localization and clear visual patterns, it requires significant processing time. Combining it with a recursive EKF loop allows for real-time tracking of voltage parameters, even in heavily distorted environments with significant background noise.

#### 2. Literature Review Table

| Parameter | Details & Metrics | Exact Location in Paper |
| --- | --- | --- |
| Author(s) & Year | P. K. Dash, M. V. Chilukuri (2004) | Page 588, Title / Meta |
| Methodology | Hybrid S-Transform (for feature extraction and localization) + Extended Kalman Filter (for recursive state tracking). | Page 588 (Abstract), Page 589 (Sec. II) |
| Dataset Used | Synthetically generated short-duration power system disturbances verified with physical hardware laboratory signals. | Page 592 (Section IV) |
| Features Extracted | Frequency contours, instantaneous amplitude parameters, and time-frequency phase properties. | Page 590 (Section II-C) |
| Classification Tech. | Rule-based classification driven by state estimation thresholds from the Kalman Filter output. | Page 591 (Section III) |
| Performance Metrics | Demonstrates precise tracking of phase angle steps and amplitude drops down to sub-cycle durations under high noise conditions. | Pages 592–595 (Figures 4–11 execution traces) |
| Limitations | Evaluating complex multi-stage transcendental equations and running recursive matrix inversions for the EKF can create processing bottlenecks during high-frequency transients. | Page 590, Section II-B |

#### 3. Dataset Summary & Generation Gist

- Dataset Specifications: The dataset covers short-duration power quality events, including voltage sags, swells, interruptions, phase shifts, and oscillatory transients. The baseline sampling configuration is designed around 1.6 kHz to 3.2 kHz. The data is generated using custom mathematical scripts, and the signals are verified using an analog-to-digital hardware link.

- Governing Generation Equations: * Standard Continuous S-Transform: $S(\tau, f) = \int_{-\infty}^{\infty} h(t) \frac{|f|}{\sqrt{2\pi}} e^{-\frac{(\tau-t)^2 f^2}{2}} e^{-i 2\pi f t} dt$, where the Gaussian window scales inversely with frequency to optimize resolution. (Page 589, Equation 1).

  - State-Space Formulation for EKF: $x_{k+1} = \Phi_k x_k + w_k$, where the measurement vector $y_k = A_k \sin(\omega k \Delta t + \phi_k) + v_k$. (Page 590, Equations 8–11).

- Python Code Logic: Define a discrete time step array $\Delta t$. Implement the recursive EKF loop by setting up your state vector $[A_k, \omega_k, \phi_k]^T$. Update the Kalman gain matrix, project the error covariance step-by-step using numpy.linalg.inv, and track how the state estimates converge during a simulated voltage drop.

# Paper 15:

### Power Quality Disturbances Detection and Classification Based on Deep Convolution Auto-Encoder Networks

#### 1. Comprehensive Executive Summary

Traditional power quality analysis usually treats feature engineering and classification as two independent, disjoint blocks. Handcrafted features can lose critical information, particularly under low signal-to-noise ratios (SNR). This paper changes the narrative by using a hybrid image processing and deep auto-encoder network approach.

Instead of passing raw 1-D time-series data directly into a network, the authors convert the 1-D voltage waveforms into 2-D grayscale grid images. They then apply a Gabor filter spatial convolution layer to perform initial macro-feature mapping. These texture maps are then fed into a Deep Convolutional Sparse Auto-Encoder (DCAE) network. Because it is a sparse framework, it forces the network to activate only a minimal percentage of its hidden nodes, compressing the image dimensions while capturing the most vital mathematical anomalies. The optimal feature weights outputted by the bottleneck layer are mapped directly to a Softmax classifier. Furthermore, the temporal pixel mapping allows the framework to determine the precise initiation and termination points of transient disturbances without relying on complex segmentation rules.

#### 2. Literature Review Table

- Author(s): Poras Khetarpal, Neelu Nagpal, Mohammed S. Al-Numay, Pierluigi Siano, Yogendra Arya, and Neelam Kassarwani

- Year: 2023

- Methodology: 1-D time-series waveform conversion to 2-D grayscale images $\rightarrow$ Spatial filtering with 2-D Gabor Filters $\rightarrow$ Feature reduction and compression using Deep Convolutional Sparse Autoencoders (DCAE) $\rightarrow$ Dense Softmax prediction.

- Dataset Used: Globally simulated dataset mapping 15 distinct classes of single and compound power quality disturbances.

- Features Extracted: Latent sparse feature vectors capturing edge intensities, spatial structural variances, and localized texture frequencies from Gabor-convolved images.

- Classification Technique: Deep Convolutional Autoencoder paired with a Softmax output layer.

- Performance Metrics: Reaches an overall classification accuracy exceeding 97.00% under severe 20 dB noise conditions.

- Limitations: Converting 1-D continuous signals into high-resolution 2-D matrices increases spatial memory storage requirements and demands high compute overhead during the initial image-generation step.

#### 3. Dataset, Equations, and Simulation Framework

The dataset contains 15 classes (comprising pure sags, swells, interruptions, harmonics, flickers, and hybrid combinations).

The Gabor filter kernel $G(x,y;\lambda,\theta,\psi,\sigma,\gamma)$ used to extract spatial features is given by:

$$G(x,y;\lambda,\theta,\psi,\sigma,\gamma) = \exp\left(-\frac{x'^2 + \gamma^2 y'^2}{2\sigma^2}\right) \cos\left(2\pi\frac{x'}{\lambda} + \psi\right)$$

Where:

$$x' = x\cos\theta + y\sin\theta$$

$$y' = -x\sin\theta + y\cos\theta$$

- $\theta$ is the orientation of the parallel stripes.

- $\lambda$ is the wavelength of the sinusoidal factor.

- $\psi$ is the phase offset.

- $\sigma$ is the sigma/variance of the Gaussian envelope.

- $\gamma$ is the spatial aspect ratio.

# Paper 16:

### Power Quality Disturbances Recognition via Lightweight Deep Learning Framework Integrated With Time-Frequency Analysis Embedded

#### 1. Comprehensive Executive Summary

Many modern deep learning architectures fail when deployed on constrained utility edge assets (like smart meters or line-mounted RTUs) due to severe memory limitations and long execution delays. This paper directly tackles this gap by presenting MEMNet, an end-to-end, highly optimized, lightweight framework explicitly designed for real-time edge embedded systems.

The core innovation is the Morlet Wavelet Time-Frequency Convolutional (MWTconv) layer. Instead of executing a fixed, pre-calculated wavelet transform as a separate preprocessing block, the MWTconv layer treats the Morlet wavelet parameters (center frequency and scale band) as trainable network weights. These parameters are updated directly via standard gradient backpropagation during training, forcing the network to evolve its own custom time-frequency filter bank tailored exactly to the target power disturbances.

To extract features across multiple scales without exponentially increasing parameter counts, the authors integrate an Efficient Multi-Scale Attention (EMA) module. This attention layer parallelizes spatial cross-dimensional pixel processing to isolate high-frequency transients and low-frequency distortions simultaneously.

#### 2. Literature Review Table

- Author(s): Juncai Song, Yuting Wang, Yuan Gao, Hong Ye, Dengzhou Fang, Xiaoxian Wang, Zhiyong Hu, and Siliang Lu

- Year: 2025

- Methodology: Adaptive Time-Frequency extraction using an active Morlet Wavelet Convolutional (MWTconv) layer $\rightarrow$ Feature refinement via an Efficient Multi-Scale Attention (EMA) framework $\rightarrow$ MEMNet deep classification.

- Dataset Used: Comprehensive synthetic dataset spanning complex single and overlapping multi-event PQDs.

- Features Extracted: Dynamically scalable time-frequency energy coefficients, shift-invariant transient edge maps, and cross-channel scale weights.

- Classification Technique: MEMNet lightweight deep neural classifier.

- Performance Metrics: Achieves a classification accuracy of 99.20% with a blistering 6.2 ms execution inference speed per sample.

- Limitations: The backpropagation optimization of wavelet parameters is highly sensitive to the chosen initial parameter seeds; bad initialization can cause the model to get stuck in local minima.

#### 3. Dataset, Equations, and Simulation Framework

The model was verified on a hardware in-the-loop setup using a Raspberry Pi 4B to prove its true embedded edge readiness.

The mathematical foundation of the adaptive Morlet wavelet function $\psi(t)$ within the trainable MWTconv layer is defined as:

$$\psi(t) = \frac{1}{\sqrt{\pi f_b}} \exp\left(-\frac{t^2}{f_b}\right) \exp(j 2\pi f_c t)$$

Where:

- $f_c$ is the center frequency of the wavelet kernel (optimized via backpropagation).

- $f_b$ is the bandwidth/bandwidth control factor (optimized via backpropagation).

# Paper 17:

### Perception of power quality disturbances using Fourier, Short-Time Fourier, continuous and discrete wavelet transforms

#### 1. Comprehensive Executive Summary

This paper serves as an extensive mathematical comparative analysis evaluating the precise physical performance and domain boundaries of four cornerstone signal processing algorithms: Fourier Transform (FT), Short-Time Fourier Transform (STFT), Continuous Wavelet Transform (CWT), and Discrete Wavelet Transform (DWT).

The authors step through each method's ability to localize specific disturbances:

- FT: Excellent for uniform steady-state harmonics, but completely blind to temporal tracking, failing to identify when a sag or swell begins or ends.

- STFT: Introduces a fixed sliding window to provide time-frequency mapping. However, it is bound by Heisenberg’s Uncertainty Principle—a wide window gives sharp frequency resolution but blurs time, while a narrow window pinpoints transient edges but destroys steady-state frequency tracking.

- CWT: Resolves this by utilizing a continuously scalable mother wavelet, yielding high time resolution at high frequencies (perfect for sharp impulses/transients) and high frequency resolution at low frequencies (perfect for tracking sags/swells).

- DWT: Breaks the signal down into discrete dyadic sub-bands, outputting Approximations (low frequency) and Details (high frequency). This offers immense computational efficiency and outputs localized energy values across separate decomposition levels to identify variations.

#### 2. Literature Review Table

- Author(s): M. S. Priyadarshini, Mohit Bajaj, Lukas Prokop, and Milkias Berhanu

- Year: 2024

- Methodology: Domain-transformation comparative study using FT vs. STFT vs. CWT vs. DWT on power signals.

- Dataset Used: MATLAB-generated mathematical disturbance models.

- Features Extracted: 3D time-frequency energy distributions, spectral components, and dyadic multi-resolution energy values.

- Classification Technique: Pure Analytical Visualization and energy threshold profiling (no black-box AI classifier used).

- Performance Metrics: Evaluated qualitatively through 3D energy-surface plots and quantitatively via multi-level sub-band energy indices.

- Limitations: The paper outlines analytical processing boundaries but does not design an automated multi-class machine learning classification engine.

#### 3. Dataset, Equations, and Simulation Framework

- Dataset Origin: Generated completely from scratch via simulation code developed in MATLAB.

- Methodology: The authors used mathematical parametric models and step functions based on standard electrical formulas to synthesize each specific voltage disturbance waveform.

- Data Specifications: * The baseline signal is modeled as a pure sinusoidal waveform with a fundamental frequency of 50 Hz and a voltage magnitude of 1 per unit (pu), spanning a duration of 0.25 seconds.

  - The final signal array for analysis contains 4,001 to 4,096 samples per waveform to optimize calculation speeds.

  - The dataset consists of generated examples across 7 categories: sag, swell, interruption, transient, harmonics, fluctuations, and flicker.

- Standardization: The variables, boundaries, and controlled equations are explicitly restricted to match the standards defined by the IEEE and the International Electrotechnical Commission (IEC).

The reference mathematical baseline for the pure fundamental voltage waveform is modeled as:

$$V(t) = A \sin(\omega t) = A \sin(2\pi f t)$$

The Continuous Wavelet Transform equation mapping the signal $x(t)$ onto the mother wavelet $\psi(t)$ is defined as:

$$CWT(a, b) = \frac{1}{\sqrt{|a|}} \int_{-\infty}^{+\infty} x(t) \psi^*\left(\frac{t - b}{a}\right) dt$$

Where:

- $a$ represents the scaling/frequency factor.

- $b$ represents the translation/time-shift parameter.

- $\psi^*(t)$ is the complex conjugate of the chosen mother wavelet function.

# Paper 18:

### Real-Time Detection and Classification of Power Quality Disturbances

#### 1. Comprehensive Executive Summary

Most classification frameworks evaluate signals offline using fixed-length historical data windows. In real-world substations, waiting for an observation window to close before making a prediction can result in catastrophic component damage. This paper introduces a highly responsive paradigm: Non-parametric, multivariate sequential multi-hypothesis testing for real-time anomaly tracking.

Instead of processing individual meters in isolation, the framework implements a multivariate cooperative detector that jointly processes streaming vectors across multiple grid nodes simultaneously. The core calculation relies on a modified Matrix Cumulative Sum (CUSUM) anomaly framework. The system constantly monitors the data stream, building a statistical profile of nominal grid conditions.

When a disturbance hits, the system updates its log-likelihood ratios sequentially, index-by-index. The moment the accumulated statistical drift crosses an analytically derived threshold, an anomaly flag is instantly thrown. This is then routed into a sequential multi-hypothesis network where the signal is matched against active profiles of various disturbance classes, finalizing the classification with minimal observation delay.

#### 2. Literature Review Table

- Author(s): Mahsa Mozaffari, Keval Doshi, and Yasin Yilmaz

- Year: 2022

- Methodology: Multi-meter Multivariate Cooperative Tracking $\rightarrow$ Sequential Log-Likelihood update parsing $\rightarrow$ Non-parametric Matrix Cumulative Sum (CUSUM) multi-hypothesis classification.

- Dataset Used: High-dimensional streaming grid data simulating concurrent single and multi-node network events.

- Features Extracted: Real-time sequential log-likelihood profiles and cross-channel correlation matrix shifts.

- Classification Technique: Sequential Multi-Hypothesis CUSUM Decision Engine.

- Performance Metrics: Minimizes detection latency (delay) to fraction-of-a-cycle scales while maintaining optimal classification accuracy.

- Limitations: Highly dynamic load changes or high steady-state background noise can cause false alarms if the baseline nominal calibration threshold isn't updated adaptively.

#### 3. Dataset, Equations, and Simulation Framework

  Dataset Origin: Synthetically generated using a MATLAB simulation framework specifically tailored to train and validate real-time pattern-recognition models.

  Methodology: The paper requires distinct training subsets for a multi-hypothesis setting (one baseline "no disturbance" set and one set per specific disturbance type). Rather than using real utility grid captures, these nominal and anomalous grid signal sequences were created mathematically to benchmark their proposed sequential multivariate detector.

  Data Specifications: * The signals track continuous voltage and current variations modeled over multi-meter frameworks (simulating joint data streams from multiple smart meters simultaneously).

- Synthetic anomalies were generated by adjusting amplitude, duration, phase transitions, and damping frequencies to mimic typical grid events.

  Standardization: The synthetic events were modeled on the benchmark mathematical formulations widely accepted in IEEE literature for testing grid anomaly detection systems.

The framework optimizes the fundamental tradeoff between false alarm rate and mean detection delay using a sequential probability ratio formulation.

The sequential multi-hypothesis decision engine updates the decision metric $S_n^k$ for each hypothesis $k$ (representing a specific disturbance type) at time step $n$ via the following recursive log-likelihood formulation:

$$S_n^k = \max \left( 0, S_{n-1}^k + \ln \frac{P(x_n | H_k)}{P(x_n | H_0)} \right)$$

Where:

- $H_0$ is the null hypothesis (representing completely clean, nominal grid operation).

- $H_k$ is the active hypothesis for disturbance type $k$.

- $x_n$ is the multivariate signal vector observation received at time step $n$.

- The event is fully classified as disturbance $k$ the exact instant $S_n^k \ge h_k$, where $h_k$ is the pre-calculated detection threshold limit.